RUNS THE TRUNK FILE COMPILER

(SHOULD BE RUN ONLY AFTER RUNNING 'CIP'

- TYPE 'HELP DATABASES' FOR A EXPLA-

NATION OF THIS FILE)

'RUN CODUMP' RUNS THE UTILITY WHICH

DUMPS COI(CENTRAL OFFICE INFORMATION)TO

THE SYSTEM LINE PRINTER

'RUN RTGDDMP' RUNS THE UTILITY WHICH

DUMPS THE ROUTINE TEST FILES TO THE

SYSTEM LINE PRINTER

'RUN DOWN' RUNS THE PROGRAM WHICH SHUTS

DOWN RAMPART APPLICATIONS PROGRAMS IN

AN ORDERLY FASHION. (DO NOT USE THIS

IT IS TO YOUR ADVNTAGE NOT TO FOR IT

WILL NOT ALLOW YOU<UNPRIV.> TO USE ANY

CMDS.)

->REFERRAL NUMBERS

REFERRAL NUMBERS ENABLE THE USER TO

QUICKLY SPECIFY A SINGLE TRUNK GROUP IN

THE SYSTEM BY ITS NUMERIC REFERRANCE.

FOR EXAMPLE, THE TRUNK GROUP 'TG123'

FROM OFFICE 'ABC' TO OFFICE 'XYZ' CAN

BE SPECIFIED BY A SINGLE NUMBER IN

RAMPART COMMANDS RATHER THAN ENTERING

THE NEAREND,FAREND, AND GROUP ID IN

THIER ENTIRETY.

NOTE: THE REFERRAL NUMBER FOR A PART-

ICULAR TRUNK GROUP MAY CHANGE IF THE DATABASE IS RE-COMPILED.

FORMAT: REF-ERRAL NUMBER

RF NUMBER

THE EASIEST WAS TO DETERMINE THE REFER-

RAL NUMBER OF A TRUNK GROUP IS TO USE

THE 'DISPLAY TRUNK' COMMAND.

->DISPLAY TRUNK

DISPLAY TRUNK(GROUPS) NEAREND<FAREND>

<GROUP^ID> OR DISPLAY TRUNK (GROUP)

REFERRAL NUMBER

DISPLAYS DATABASE INFORMATION ON THE

TRUNK GROUP(S) SPECIFIED. INCLUDED IS

REFERRAL NUMBER, NEAREND, FAREND,

GROUP ID, NUMBER OF TRUNKS, SYSTEM

TRUNK NUMBERS, AND LIMITS DATA.

O  IF ONLY 'NEAREND' IS SPECIFIED, ALL

TRUNK GROUPS ORIGINATING FROM THE

OFFICE SPECIFIED ARE DISPLAYED

O  IF 'NEAREND' AND 'FAREND' ARE

SPECIFIED, ALL GROUPS BETWEEN THESE

TWO OFFICES ARE DISPLAYED

O  SPECIFYING 'NEAREND', 'FAREND', AND

'GROUP^ID',OR SPECIFYING THE REFERAL

NUMBER UNIQUELY DESCRIBES A SINGLE

TRUNK GROUP.

->TRUNK

TRUNK NUMBERS

RAMPART USES TWO DIFFERENT TRUNK

NUMBERING SYSTEMS; NAMELY, USER TRUNK

NUMBERS AND SYSTEM TRUNK NUMBERS. USER

TRUNK NUMBERS ALWAYS START AT ONE, AND

END AT THE NUMBER OF TRUNKS IN THE

GROUP. SYSTEM TRUNK NUMBERS ARE THE

NUMBERS USED TO ACCESS TRUNKS ON THE

SWITCH AND MAY RANGE FROM 0 TO 65000.

FOR COMMANDS WHICH REQUIRE TRUNK

NUMBERS,THE '/SY' SWITCH SHOULD BE

ADDED TO THE COMMAND LINE WHEN REFER-

ENCING SYSTEM TRUNK NUMBERS.

->DATABASE

RAMPART DATABASE OVERVIEW

THE RAMPART USER DATABASE IS COMPRISED

OF THE FOLLOWING TWO FILES:

(1) CENTR. OFFICE DESCRIPTOR(COD) FILE-

THE EQUIPMENT LOCATED IN THE CENTRAL

OFFICE KNOWN TO THE SYSTEM. IT CONTAINS

SUCH INFO. AS COMMON LANGAUGE CODES,

LITERAL CENTRAL OFFICE NAMES, ROTL TYPE

AND ACCESS NUMBER, TESTLINE TYPES AND

ACCESS NUMBERS,ECT.

(2) TRUNK SOURCE FILE- THIS FILE

DESCRIBES THE TRUNKS LINKING <CO>IN

THE <COD>FILE. IT CONTAINS SUCH INFO AS

TRUNK GROUP ID, NUMBER OF TRUNKS IN

GROUP, LIMITS DATA, TRANSMIT LEVEL,

SIGNALLING,ROUTINE TEST INTERVAL,ECT.

THESE FILES CAN BE UPDATED BY THE USER

SOURCE EDITOR (USE) PROGRAM (INVOKED BY

THE CMDS 'EDIT' & 'RUN USE').

[DATABASE SUB-CMDS]

EXAMPLE

COD        CEN-TRAL^OFFICE

TRUNK

->DATABASE COD

THIS FILE CONTAINS INFORMATION ON THE

EQPT. LOCATED IN <CO> KNOWN TO RAMPRT.

FOR DETAILED INFORMATION ON ITS CONT.,

RUN THE USER SOURCE EDITOR,SPECIFY COD

FILE EDIT, INVOKE CHANGE MODE ON AN

EXSISTING CENTRAL OFFICE RECORD, AND

USE THE HELP FACILITY BUILT INTO THE

EDITOR.

DATABASE EXAMPLE

----------------

,-------------------,

, #3 EAX ROTL       ,

, (TEST PORT ACCESS ,

,  NUMBER = 5551234),

-------------------

:

:

:         :::::::::::::

:::::::::105 RESPONDER:

:::::::::::::

->CHECK

THIS COMMAND CAUSES A RAMPART TEST PORT

TO TO INITIATE A SELFCHECK SEQUENCE AND

RETURN THE SELFCHECK DISPOSITION TO THE

REQUESTING TERMINAL.

FORMAT: CHECK <PORT> PORT # (OR ALL).

->EDIT

INVOKES THE USER SOURCE FILE EDITOR

PROGRAM WHICH IS USED TO UPDATE THE

RAMPART DATA BASE. THIS PRG. HAS ITS

OWN HELP FACILITY.

FOR INFO. ON THE RAMPART DATABASE IN

GENERAL, TYPE 'HELP DATABASE'.

#

EXPLANATION OF THE CONTENTS OF THIS

FILE)

'RUN TAC'## # #