==)--- P TO PAUSE S TO STOP ---(==

#          !WHACKOLAND!WHACKOLAND!

W   HACKING RAMPART   W

H       SYSTEMS       H

A                     A

C  PART II.....       C

K                     K

O      IN DEPTH       O

L   DESCRIPTIONS OF   L

A      COMMANDS       A

N         &           N

D      SUB-CMDS       D

!WHACKOLAND!WHACKOLAND!

THIS IS THE SECOND PART OF IN-DEPTH

COMMAND & SUB COMMAND'S. READ PART I

OF HACKING RAMPART SYSTEMS BEFORE THIS.

->PUT

THESE COMMANDS ALTER THE CONFIGURATION

OF RAMPART EQUIPMENT

FORMAT: PUT (OR PLACE) QUALIFIER IN

(OR OUT OF SERVICE)

] PUT(OR PLACE) SUB CMDS [

TESTLINE

PORT

ROTL

->PUT TESTLINE

PUT TYPE TESTLINE (AT) CENTRAL^OFFICE

IN (OR OUT OF SERVICE)

MAKES A TESTLINE AVAILABLE OR UN-

AVAILABLE FOR TESTING BY RAMPART (HAS

NO PHYSICAL EFFECT ON THE EQUIPMENT

ITSELF.)

O   POSSIBLE TESTLINE TYPES: 100, 102,

103, 104, 105

O   CENTRAL^OFFICE CAN BE A COMMON

LANGUAGE CODE OR A LITERAL CENTRAL

OFFICE NAME (CON).

->PUT PORT

PUT PORT NUMBER (OR ALL) QUALIFIER

ALTERS THE CONFIGURATION OF RAMPART

TEST PORTS.

->PUT PORT SUB CMDS

IN     OUT

PBX

WATS

DTMF   DP

->PUT PORT NUMBER (OR ALL) <TO> PBX

(OR NO PBX)

ENABLES OR DISABLES DIALING '9' AND

WAITING FOR DIAL TONE BEFORE DIALING

ROTL ACCESS NUMBERS.

->PUT PORT WATS

PUT PORT NUMBER (OR ALL) <TO> WATS (OR

NO WATS)

ENABLES OR DISABLES DIALING '8' AND

WAITING FOR DIAL TONE BEFORE DIALING

ROTL ACCESS NUMBERS

->PUT PORT DTMF

PUT PORT NUMBER (OR ALL) <TO DTMF>

(OR NO DP) CAUSES THE SPECIFIED PORT(S)

TO USE DIAL PULSE OUTDIALING

->DISPLAY

THESE COMMANDS RETURN A DISPLAY TO THE

USER OF SYSTEM PARAMETERS AND STORED

TEST RESULTS.

FORMAT: DISPLAY (OR SHOW) QUALIFIER

ABBREVIATIONS: DP,?

->DISPLAY SUB CMDS

ROTL

TESTLINE

PORT

RES-ULTS

DEMAND

TRUNK

RUT

->TERMINAL

TERMINL <(IS) TERMINAL@TYPE>

DISPLAYS THE DEVICE NUMBER OF THE

TERMINAL CURRENTLY IN USE & OPTIONALLY

DEFINES THE TERMINAL USER'S TERMINAL

TYPE.

VALUES FOR 'TERMINAL@TYPE':CRT, NO CRT,

VT100, VT101, VT131, VT125 (USE 'NO

CRT' FOR HARD COPY TERMINALS. NO CRT IS

THE DEFAULT WHEN LOGGING IN ON REMOTE

TERMINALS)

NOTE: THE CORRECT TERMINAL TYPE MUST BE

DEFINED TO ENABLE CERTAIN PLOTS AND

GRAPHS ON THE RAMPART SYSTEM.

->TEST

THIS CLASS OF CMDS ENABLES THE RAMPART

USER TO PERFORM DEMAND TESTS. IF ALL

TEST PORTS ARE IN USE, DEMAND REQUESTS

ARE PUT INTO THE DEMAND QUEUE & TESTED

IN 'FIRST IN  - FIRST OUT' ORDER.

FORMAT: TEST TESTLINE (FROM) NEAREND

(TO) FAREND (ON GROUP) GROUP ID

<(ON TRUNKS)TRUNK NUMBERS> >PERFORMING

TESTS>SWITCHES>

TEST TESTLINE REFERRRAL NUMBER <(ON

TRUNKS) TRUNK NUMBERS><(PERFORMING)

TESTS> <SWITCHES>

O  'TESTLINE' DEFAULTS TO 105 IF NOT

SPECIFIED.

O  'TRUNK NUMBERS' DEFAULT TO ENTIRE

TRUNK GROUP IF NOT SPECIFIED

O  'TESTS' DEFAULT TO 2-WAY LOSS AND

NOISE.

->TEST SUB CMD'S

NEAREND    FAREND

TESTLINE

TESTS

SWITCHES

TRUNK@NUMBERS

REP-EAT

THE TESTS THE THE RAMPART SYSTEM IS

CAPABLE TO PERFORM ARE:

'L'=TWO WAY LOSS AT 1004 HZ

'L4'=TWO WAY LOSS AT 404 HZ,-16 DB

'L10'=TWO WAY LOSS AT 1004 HZ, -16 DB

'L28'=TWO WAY LOSS AT 2804 HZ, -16 DB

'N'=NEAR TO FAR NOISE

'RN'=FAR TO NEAR NOISE

'NT'=TWO WAY NOISE WITH TONE

'ERL'=ECHO RETURN LOSS

'SRL'=SINGING RETURN LOSS

'SRLHI'=HIGH SINGING RETURN LOSS

'ALL' PERFORMS ALL TESTS AVAILABLE ON

FAREND RESPONDER SPECIFIED.

AVAILABLE SELFCHECKS:

'LSC', 'LSC4', 'LSC10', 'LSC28', 'NSC'

'RNSC','NTSC', 'ERLSC', 'SRLSC','SHISC'

->REPORT RUT

ALLOWS THE USER TO SPECIFY REPORTING ON

ALL THE SPECIFIED RUT PW IN THE DB.

IF THE RUT PW IS NOT SPECIFIED IN THE

CMD LINE, THE USER IS PROMPTED FOR THE

RUT PW TO USE IN REPORT SELECTION

(RUT IS A ACRONYM FOR REMOTE USER

TERMINAL)

EXAMPLE: 'REPORT RUT AAA'

'REPORT RUT' (PROMPTS FOR RUT

PW)

-> REPORT RUT TESTS

ORIGINATING TERMINAL TT4:

DEMAND TEST-TRUNK GROUP DALTER20## # # # # # # # # #<#G#>#H#a#c#k#i#n#g# #A#r#