!WHACKOLAND!WHACKOLAND!          W   HACKING RAMPART   W          H       SYSTEMS       H          A                     A          C  PART II.....       C          K                     K          O      IN DEPTH       O          L   DESCRIPTIONS OF   L          A      COMMANDS       A          N         &           N          D      SUB-CMDS       D          !WHACKOLAND!WHACKOLAND! THIS IS THE SECOND PART OF IN-DEPTHCOMMAND & SUB COMMAND'S. READ PART IOF HACKING RAMPART SYSTEMS BEFORE THIS.   ->PUTTHESE COMMANDS ALTER THE CONFIGURATIONOF RAMPART EQUIPMENTFORMAT: PUT (OR PLACE) QUALIFIER IN(OR OUT OF SERVICE)  ] PUT(OR PLACE) SUB CMDS [TESTLINEPORTROTL   ->PUT TESTLINEPUT TYPE TESTLINE (AT) CENTRAL^OFFICEIN (OR OUT OF SERVICE)MAKES A TESTLINE AVAILABLE OR UN-AVAILABLE FOR TESTING BY RAMPART (HASNO PHYSICAL EFFECT ON THE EQUIPMENTITSELF.)O   POSSIBLE TESTLINE TYPES: 100, 102,  103, 104, 105O   CENTRAL^OFFICE CAN BE A COMMON  LANGUAGE CODE OR A LITERAL CENTRAL  OFFICE NAME (CON).   ->PUT PORTPUT PORT NUMBER (OR ALL) QUALIFIERALTERS THE CONFIGURATION OF RAMPARTTEST PORTS.   ->PUT PORT SUB CMDSIN     OUTPBXWATSDTMF   DP   ->PUT PORT NUMBER (OR ALL) <TO> PBX(OR NO PBX)ENABLES OR DISABLES DIALING '9' ANDWAITING FOR DIAL TONE BEFORE DIALINGROTL ACCESS NUMBERS.   ->PUT PORT WATSPUT PORT NUMBER (OR ALL) <TO> WATS (ORNO WATS)ENABLES OR DISABLES DIALING '8' ANDWAITING FOR DIAL TONE BEFORE DIALINGROTL ACCESS NUMBERS   ->PUT PORT DTMFPUT PORT NUMBER (OR ALL) <TO DTMF>(OR NO DP) CAUSES THE SPECIFIED PORT(S)TO USE DIAL PULSE OUTDIALING   ->DISPLAYTHESE COMMANDS RETURN A DISPLAY TO THEUSER OF SYSTEM PARAMETERS AND STOREDTEST RESULTS.FORMAT: DISPLAY (OR SHOW) QUALIFIERABBREVIATIONS: DP,?   ->DISPLAY SUB CMDSROTLTESTLINEPORTRES-ULTSDEMANDTRUNKRUT   ->TERMINALTERMINL <(IS) TERMINAL@TYPE>DISPLAYS THE DEVICE NUMBER OF THETERMINAL CURRENTLY IN USE & OPTIONALLYDEFINES THE TERMINAL USER'S TERMINALTYPE.VALUES FOR 'TERMINAL@TYPE':CRT, NO CRT,VT100, VT101, VT131, VT125 (USE 'NOCRT' FOR HARD COPY TERMINALS. NO CRT ISTHE DEFAULT WHEN LOGGING IN ON REMOTETERMINALS)NOTE: THE CORRECT TERMINAL TYPE MUST BEDEFINED TO ENABLE CERTAIN PLOTS ANDGRAPHS ON THE RAMPART SYSTEM.   ->TESTTHIS CLASS OF CMDS ENABLES THE RAMPARTUSER TO PERFORM DEMAND TESTS. IF ALLTEST PORTS ARE IN USE, DEMAND REQUESTSARE PUT INTO THE DEMAND QUEUE & TESTEDIN 'FIRST IN  - FIRST OUT' ORDER.FORMAT: TEST TESTLINE (FROM) NEAREND(TO) FAREND (ON GROUP) GROUP ID<(ON TRUNKS)TRUNK NUMBERS> >PERFORMINGTESTS>SWITCHES>TEST TESTLINE REFERRRAL NUMBER <(ONTRUNKS) TRUNK NUMBERS><(PERFORMING)TESTS> <SWITCHES>O  'TESTLINE' DEFAULTS TO 105 IF NOT  SPECIFIED.O  'TRUNK NUMBERS' DEFAULT TO ENTIRE  TRUNK GROUP IF NOT SPECIFIEDO  'TESTS' DEFAULT TO 2-WAY LOSS AND   NOISE.   ->TEST SUB CMD'SNEAREND    FARENDTESTLINETESTSSWITCHESTRUNK@NUMBERSREP-EAT  THE TESTS THE THE RAMPART SYSTEM ISCAPABLE TO PERFORM ARE:    'L'=TWO WAY LOSS AT 1004 HZ   'L4'=TWO WAY LOSS AT 404 HZ,-16 DB  'L10'=TWO WAY LOSS AT 1004 HZ, -16 DB  'L28'=TWO WAY LOSS AT 2804 HZ, -16 DB    'N'=NEAR TO FAR NOISE   'RN'=FAR TO NEAR NOISE   'NT'=TWO WAY NOISE WITH TONE  'ERL'=ECHO RETURN LOSS  'SRL'=SINGING RETURN LOSS'SRLHI'=HIGH SINGING RETURN LOSS'ALL' PERFORMS ALL TESTS AVAILABLE ONFAREND RESPONDER SPECIFIED.AVAILABLE SELFCHECKS: 'LSC', 'LSC4', 'LSC10', 'LSC28', 'NSC''RNSC','NTSC', 'ERLSC', 'SRLSC','SHISC'    ->REPORT RUTALLOWS THE USER TO SPECIFY REPORTING ONALL THE SPECIFIED RUT PW IN THE DB.IF THE RUT PW IS NOT SPECIFIED IN THECMD LINE, THE USER IS PROMPTED FOR THERUT PW TO USE IN REPORT SELECTION(RUT IS A ACRONYM FOR REMOTE USERTERMINAL)EXAMPLE: 'REPORT RUT AAA'         'REPORT RUT' (PROMPTS FOR RUTPW)   -> REPORT RUT TESTSORIGINATING TERMINAL TT4:DEMAND TEST-TRUNK GROUP DALTER2004,CINDSROTL TO DALCTSSROTL EML @NE @FE 0.0Q1 LOSS LIMIT 2.0  Q2 LOSS LIMIT 04.0Q1 ERL LIMIT 00.   Q2 ERL LIMIT   00.Q1 NOISE LIMIT 44. Q2 NOISE LIMIT 47.Q1 NTONE LIMIT 44. Q2 NTONE LIMIT 47.PORT 2, 20-NOV-85 00:21:00 REF PFX:100TRANSMISSION 105 TEST (RESULTS STORED)ROTL ACCESS: 3217006TRUNK1/ 6001 > 00:22:06 KP 05200460010003214007 STL        F-N -06.1  N-F -00.1RN       F-N *  *  THESE TESTS ARE BEING ROUTED THROUGHEITHER THE WATS OR PBX, OR NO PBX. YOUCAN TOGGLE THESE.          !WHACKOLAND!WHACKOLAND!          !                     !          T LOOK FOR PART III.. T  >T<     W   HACKING RAMPART   W          C    CL 1, CL2, CL3   C          B     MAIN SYSTEM     B          !   (314) 256 8220    !          !WHACKOLAND!WHACKOLAND!#################################DOWNLOADED FROM P-80 SYSTEMS.....                           #################################################################