Unauthorised Access UK  0636-708063  10pm-7am  12oo/24oo

HOW TO DIAL OUT ON A UNIX SYSTEM.

---------------------------------

This document makes the assumption that you are currently logged on to the

system and are sitting in a shell environment.

1. First of all we need to locate the L-devices file.

It should be found in the /usr/lib/uucp directory,

but in case it isn't typing:

find / -name L-devices -print

will show you where it is.

If you can't find it then don't worry as we can get

around it, only it will take a bit of trial and error.

2. If you found the L-devices file then we need to list

it by typing:

cat L-devices

If it runs off the screen then type:

cat L-devices | more

This will page the output - space displays the next

page and return will show the next line while q quits.

This file shows us to which serial line (port) the

modems (ACU's) are connected, it also shows when they

can be called and the baud rate.

We are interested in the serial line and the baud rate.

Choose a line with your desired speed and make a note of

the serial line. The speed is shown as 2400,1200,300 etc.

and the serial line as ttynn where nn is a number.

3. If you couldn't find/list the L-devices file then type:

who am i

This will show which serial line you are on, and as you

are on a modem then it's a fair bet that the others are

not too far away. e.g. If you are on line tty07 then

there's a good chance of a modem being on tty06,tty08 or

thereabouts.

4. Now we need to make a direct connection to the modem by

typing:

cu -sbaud -l/dev/ttynn dir

where baud and ttynn are your desired speed and serial

line respectively.

If you couldn't find/list the L-devices file then this is

where the trial and error I told you about comes in.

When you get it right it should come up with 'Connected'.

5. Now we are talking directly to the modem. As a precaution

at this point I suggest saving the modem's current config

by typing:

AT&W

Don't worry if you can't see what you are typing as it is

probably in quiet mode with echo off.

Now restore the factory default settings by typing:

AT&F

Now you can set up the modem as you require it, just as

you would with your own and use it as normal.

When you have finished type:

ATZ

to restore the modem back to it's initial state, then

type:

~.

It should come up 'Disconnected' and you should now be

back in your shell.

-------------------------------

It is probably best to try and log on to a RACE system on an 0800 number

as then it won't cost you a penny to call your favourite BBS's.

If you have any problems or need any UNIX information then send mail to

me on HackerNet and I'll see what I can do for you.

Wishing you all many, many, happy hours of hacking - THE ANALYST.

Downloaded From P-80 International Information Systems 304-744-2253