==)--- P TO PAUSE S TO STOP ---(==

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`

Basic Signaling, PT I

{C} 1987 Asmodeus Rex OOTR/cDc

Transcribed From 1986 Bell Tech

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`

This is the start of a sequence of info that someof you may or may now know..

if you know it all, tell me and i will resort to something else..

Basic Signaling

4.1 General

Signaling is the means used to establish and control telephone

calls. It includes signals from terminals (station signaling), to and from

switching centers (register signaling), and also signals between switching

centers (line signaling). Signals are carried by the line and trunk

transmission equipment and the switching system which convay information and

commands to and from the various parts of the system and operating

administration signals.

The modes of signaling which will be summarized include the following

types..

4.1.1 Direct Current Signaling

In the direct current signaling method (loop-disconnect signaling), a

signaling code is derived from the duration and direction of the current

flowing through a loop. This loop includes the customer's telephone and the

line transmission equipment and it's switching center interface. Direce

current loop signaling is also used with trunk transmission equipment of the

wire type.

An alternative methd of direct current signaling uses only one of the

pair of wires and is called leg signaling

4.1.2 Alternating Current Signaling

Alternating current signaling is based on signals of different

frequency either in the same bandwidth (in band) as the speech transmission

path (300 to 3400 Hz) or at a lower, <300 Hz, or at a higher, >3400 Hz,

frequency (out band).

4.1.3 Digital Signals

Digital signals take the form of a series of successive pulses, binary

signals, which are coded to produce a signaling format. Digital signals may

occupy a portion of one of the time slots used for the transmission of speech

(in slot) or may used a dedicated time slot (our slot).

4.1.4 Signaling Path

Signaling may use the same transmission path as the speech or datd

signals or it may use a seperate channel. The first case is called Channel

Associated Signaling, the second case Common Channel Signaling.

Common Channel Signaling. Shows the principle of operating for common

channel signaling. The signalin function for both line and register signaling

is removed from the path carrying the transmission signals. In lieu of this a

series of messages indicating the originating and terminating trunk

identities, together with the relevant address or line signals, is sent over a

seperate dedicated channel. This information is processed in the system

controls at either end and decoded to produce the relevant actions. There are

two such systems in use, or planned for use: CCITT System #6, which uses an

analog signaling in band method by the use of data modems, and CCITT System

#7, which uses pulse code modulation (PCM) coded signals operating at 64,000

BPS.

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\

Basic Signaling Pt II

{C} 1987 Asmodeus Rex - cDc

OOTR/PAWW/MBI/MCI

Transcribed From 1986 Bell

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\

This File Was First Left On P-80, All Credit Of This File Must Be Left to

P-80 When Posted On Other Systems..    - Rexxy

4.1.5 Signalin over a Carrier System

The signaling path may consist of wire conductors or it may be derived

from a carrier system. This provides a dedicated derived channgd from one of

the 'n' inputs to one of the 'n' outputs. It uses either frequency

multiplexing or pulse code modulation to multiplex the channels.

4.1.5.1 Frequency Division Carrier

In the case of frequency division a number of channels are each

allocated a por4ion of the bandwidth of the carrier system. Each portion is

multiplexed with the carrier frequency by thE multiplexing equipment at the

send end. It is demultiplexed and filtered to select the relevant channel at

the recieve end to restore the speech wave form.

4.1.5.2 Pulse Code Modulation Carrier

In the case of pulse code modulation a number of channels are each

allocated a time slot which has a fixed recurring position in a frame of time

slots. The analog input to each channel is sampled and coded at the send side

and interleaved with the other time slots by the multiplexing equip. The

channel is extracted at the receive en$ and demultiplexed back to analog.

Signaling systems are designed for the various transmission modes and

means.

4.2 Station Signaling

Station signaling controls the transmission path between the terminal

and the exchange and provides address information. It is used for the duration

of th call. Address signaling is used to set the call up and to initiate

certain kinds of services, some of which are described later on..

4.2.1 Signals From Telephone

Present-day telephones have the following signaling elements.

o  A hook switch which completes the loop circut when the telephone

handset is lifted. This is used to originate or answer a call.

Replacement of the handset opens the loop and terminates the call.

o  A calling device, which is either a rotary dial or a key pad. to

produce the address signals, henceforth the station signals

address information to the switching system center register

using either dial pulses or DTMF pulses.

o  An alerting device, usually a bell, to indicate a call terminating

on the line.

4.2.1.1 Rotary Dial

The calling device which is in most common us% as The rotary dial. The

rotary dial produces dial pulse (loop disconnect) signals.

This calling arangement uses the standard telephone dial. rotary type,

usuallt having a pulsating rate of 10 impulses per second. (IPS). However, 20

IPS dials are used in certain countrys. Dial pulsing utilizes a train of one

to 10 pulses, each pulse consisting of a momentary opening of the signaling

loop. The digit to be dialed is selected by rotating the dial manualy to the #

selected and releasing the dial produces a requisite pulse train.

The loop disconnect signals in general use, these include switch hook

or equivalant signals.

4.2.1.2 Loop-Disconnect Station Signaling

A calling line originates a call by a request for service signal which

)s produced when the telephone handset is removed. This signal is detected by

the line circut which associates a callin' device detector and a register.

Dial tone is returned to the caller, who then dials the address of the

requested party. The register stores the dialed units and hence accumulates

the address information. The line circut register and calling device deteator

respond to the following loop-disconnect signals.

'Closed Loop Signals' These can indicate a request for service or the

end of a train of digits. IN the first use the signal must persist for a

minimum of 10Msec. A signal occuring after request for service must persist

for a minimum of 180Msec. duration to be recognized as the end of a series of

pulses corresponding to a digit. (Interdigit Signal).

If the signal occurs after all digits have been recieved then it is a

control signal.

'Open Loop Signals' An open-loop signal can either be the beginning of

a dial pulse or at the beginning of a disconnect or 'hook-switch flash'

signal. If the signal is the begining of a dial pulse it will be followed by a

closed loop condition within 180 Msec. If the signal is the beginning of a

"hook switch flash" it will be follwed by a closed loop condition within 1 or

2 seconds. Services using "hook-switch flash" are those services which require

the connection of functional units to an established call without

disconnecting the established connection. Hence, depending on the class of

service of the caller, a sustained open-loop condition which occurs after a

closed loop must last longer than 180 Msec or 2 Sec to be recognized as a

disconnect signal..

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`

Basic Signaling - Pt III

{C} 1987 Asmodeus Rex - cDc/OOTR

AEUA/MCI/MBI/PAWW

Transcribed From 1986 Bell

\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`\`

4.2.1.3 Tone Signaling from Station

The advent of low-cost electronic components has made the use of

key-controlled oscillators as calling devices feasible. The generic term is

'Dual Tone Multi Frequency' (DTMF). This calling device has push-button keys

on the telephone innstruments which transmit two frequencies when one key is

pressed. Signals are those developed by the 'Bell System' for tuch-tone

service, which is a registerd trademark of Western Electric Co.

Key plates in normal use have only 10 or 12 of the 16 possible keys,

but some systems should be capable of responding to all 16 codes for future

applications.

4.2.1.4

Registers serving lines are designed to respond to either dial pulses

or DTMF signals. This uses serarate process for dial pulse and DTMF but the

resulting digit (whether it was determined by counting pulses or decoding

voice frequency signals) is coded in the system code (usually binary-coded

decimal) and stored.

4.2.1.5 Binary Coded Decimal

This coding method uses for bits per decimal digit. The first ten of

the 15 possible combinations are used. The four bits have the most significant

bit value (eight) on the left hand side and numbers are coded by summing the

bit values; EX: Decimal value 1 is coded as 0001 and decimal value 10 is coded

1010. This method, which separately codes each decimal digit, has advantages

in analyzing the digits dialed by th caller and relating them to the numbering

plan.

4.2.2 Signals To Telephones

The signals to the telephone of the calling party consist of tone

signals, or messages, to the caller. These direct the caller's actions or

advise him or her of the progress of the call. The frequencies and cadences if

these tones may vary from country to country, but the CCITT is recommending

the use of standard tones. This has become increasingly important with the

introduction of direct international dialing. Inaddition to this there are

signals which alert a caller customer. Another signal informs a customer that

the telephone handset has been left of the hook, without dialing, for a

predertimed period.

Call These BBS's

[The Alcazar Elite..............................................401/789-0094]

[Sanctuary North Private........................................507/288-9572]

Downloaded from P-80 Systems.....

Originally Displayed on P-80 Systems.