The following info comes from a leaflet entitled 'FREEFONE':

"British Telecoms recent record profits and continuing appalling

service have prompted the circulation of  this  information.  It

comprises a method of making telephone calls free of charge."

DIAGRAM:                             PARTS REQUIRED:

O----o-------o/  o------o----O       S1  =  SPST Switch

L    !        S1        !    T

I    o--------|<--------o    O       D1  =  1N4004

N    !        D1        !

E    o--XXXX------>|----o    T       D2  =  L.E.D. (TIL209 etc)

!   R1       D2    !    E

I    o--------||--------o    L       C1  =  1uF, 250vac non elec

N             C1             E

O----------------------------O       R1  =  2K8 resistor

OPERATION:

This circuit inhibits charging for incoming calls only.  When  a

phone is answered, there is normally approximately 100ma DC loop

current, but only 8ma or so is nessecary to polarise the mic  in

the handset. Drawing  only  this  small  amount  of  current  is

sufficient to fool BT's ancient 'Electronic Meccano'.

Its extremely simple. When ringing, the  polarity  of  the  line

reverses so D1 effectively answers the call when the handset  is

lifted. When the call is established, the line polarity  reverts

and R1 limits the loop current while D2 acts as an indicator  to

show when the circuit is in  use.  C1  ensures  that  speech  is

unaffected. S1 returns the telephone to normal when closed.

With the phone on the hook this circuit is undetectable  at  the

exchange and/or on the line. The switch should be closed to make

outgoing calls and when receiving calls from the operator.

The method illustrated here uses the same technology that BT use

on their 0800 exchange to obtain free incoming calls. (Try  this

number as a demo: 0800 800 800.)

*****DO NOT DIVULGE YOUR SOURCE OF THIS INFORMATION*****