==)--- P TO PAUSE S TO STOP ---(==

#

The Phreaker's Guide

to Loop Lines

by  Lefty Carlson

&

Landau Ltd

_______________________________________

Revised: April 5, 1985

A loop is a wonderous device which

the telephone company created as test

numbers for telephone repairmen when

testing equipment.  By matching the

tone of the equipment with the tone of

the loop, repairmen can adjust and test

the settings of their telephone

equipment.

A loop, basically, consists of two

different telephone numbers.  Let's

use A and B as an example.  Normally

if you call A, you will hear a loud

tone (this is a 1004 hz tone), and if

you call B, the line will connect, and

will be followed by silence.

This is the format of a loop line.

Now, if somebody calls A and someone

else calls B--Viola!--A and B loop

together, and one connection is made.

Ma Bell did this so repairmen can

communicate with each other without

having to call their own repair ofnice.

They can also use them to exchange programs, like for ANA or Ringback.

Also, many CO's have a "Loop Assign}ent Center".  If anyone has any information on these centers please tell me.

Anyway, that is how a loop is

constructed.  From this information,

anyone can find an actual loop line.

Going back to the A and B example,

Note: the tone side and the silent side

can be either A or B.  Don't be fooled

if the phone company decides to

scramble them around to be cute.

As you now know, loops come in pairs

of numbers.  Usually, right after each

other.

For example: 817-972-1890

and

817-972-1891

Or, to save space, one loop line can

be written as 817-972-1890/1 (according to the Landau Shorthand For Numbers.

This is not always true.  Sometimes,

the pattern is in the tens or hundreds,

and, occaisionally, the numbers are

random.

In cities, usually the phone company

has set aside a phone number suffix

that loops will be used for.  Many

different prefixes will correspond

with that one suffix.

In Arlington, Texas, a popular

suffix for loops is 1893 and 1894, and

a lot of prefixes match with them to

make the number.

For Example:  817-460-1893/4

817-461-1893/4

817-465-1893/4

817-467-1893/4

817-469-1893/4

...are all loops...

or a shorter way to write this is:

817-xxx-1893/4

xxx= 460, 461, 465, 467, 469

Note: You can mix-and-match a

popular suffix with other prefixs in a

city, and almost always find other

loops or test numbers.

Note: For Houston, the loop suffixes are 1499 and 1799.  And for Detroit it's 9996 and 9997.

When there are a large number of

loops with the same prefix format,

chances are that many loops will be

inter-locked.  Using the above example

of Arlington loops again, (I will write

the prefixes to save space) 460, 461,

and 469 are interlocked loops.  This

means that only one side can be used at

a given time.  This is because they are

all on the same circuit.

To clarify, if 817-461-1893 is

called, 817-460 and 469-1893 cannot be

called because that circuit is being

used.  Essentialy, interlocked loops

are all the same line, but there are a

variety of telephone numbers to access

the line.

Also, if the operator is asked to

break in on a busy loop line he/she

will say that the circuit is

overloaded, or something along those

lines.  This is because Ma Bell has taken the checking equipment off the line.  However, there are still many rarely used loops which can be verfied and can  have emergency calls taken on them.

As you have found out, loops come in

many types.  Another type of loop is a

filtered loop.  These are loop lines

that the tel co has put a filter on, so

that normal human voices cannot be

heard on either line.  However, other

frequencies may be heard.  It all

depends on what the tel co wants the

loop to be used for.  If a loop has

gotten to be very popular with the

local population or used frequently for

conferences, etc. the tel co may filter

the loop to stop the unwanted

"traffic".  Usually, the filter will be

removed after a few months, though.

The wise and knowledgeable phreaker

can use loops in many ways.  If there

is a person he/she wants to talk to,

but does not want to give them their

home telephone number, then they can

meet on a loop.  Conferences can be set

up on loops.  Collect calls can be

accepted on loops.  If a loop is

unmonitered, then it can be called free

from a pay phone (this has many good

uses if you think about it).

Therefore, loops can be a powerful tool

if used correctly, and not abused.

They can make a good phreak's life a

lot easier.

I hope that this tutorial is of use

to everyone who reads it, and was

enjoyable.  We, here at the Landau Ltd

Mansion, enjoyed writing it.  Look for

more articles, files, and tutorials

from myself, Lefty Carlson, and Landau

Ltd in the future.

PS: Look out for a part 2 of this series.  Of which, I plan it to include as large a list of working loops as possible.  I'm not exactly sure how to do it, but if anyone has any ideas please leave me mail.  Remember: Comments, critisisms, critiques, or er criteria are always welcome!

---> The End <---

_______________________________________

R. 1985/LL

#

LIBRARY MENU #1

)