==)--- P TO PAUSE S TO STOP ---(==

#

/-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-\

!          Bell Telephone Info        !

!                                     !

!  Courtesy of:                       !

!                                     !

!       The Lost City of Atlantis     !

!                                     !

!           <215>-<844>-<8836>        !

!                                     !

!                                     !

\-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-/

***************************************

BRIDGING HEADS, RESIDENTIAL AND

BUSINESS MULTILINE DISTRIBUTION BOXES,

LINE AND TRUNK SPLITTERS, AND

OTHER BELL SYSTEM WIRE TERMINATIONS.

- HOW TO USE, AND/OR ABUSE THEM -

(INCLUDING A TUTORIAL ON BASIC TELE-

PHONE EAVESDROPPING TECHNIQUES.)

***************************************

WRITTEN BY :   PHUCKED

AGENT     07/29/84

04

REVISED FOR LOD!  10/84

CURRENTLY REVISION II

***************************************

IN THIS ARTICLE, I WILL FIRST

BESCRIBE THE TERMINATION, WIRING, AND

TERMINAL HARDWARE MOST COMMONLY USED

IN THE BELL SYSTEM, AND I WILL INCLUDE

A SECTION ON METHODS OF USING THEM.

***************************************

-------------

LOCAL NETWORK

-------------

THE LOCAL TELEPHONE NETWORK

BETWEEN THE CENTRAL OFFICE/EXCHANGE AND

THE TELEPHONE SUBSCRIBERS CAN BE

BREIFLY DESCRIBED AS FOLLOWS:

FROM THE CENTRAL OFFICE (OR

LOCAL EXCHANGE) OF A CERTAIN PREFIX

(ES), UNDERGROUND AREA FEEDER TRUNKS

GO TO EACH AREA THAT HAS THAT PREFIX.

(USUALLY MORE THAN ONE PREFIX PER AREA)

AT EVERY FEW STREETS OR TRACT AREAS,

THE UNDERGROUND CABLES SURFACE. THEY

THEN GO TO THE SECONDARY TERMINATION,

(THE AERIAL TELEPHONE FEEDER CABLE)

(OR BACK UNDERGROUND, DEPENDING ON THE

AREA) AND THEN TO THE SUBSRIBERS HOUSE

(OR IN THE CASE OF AN APARTMENT

BUILDING OR MUTLILINE BUSINESS, TO

A SPLITTER OR DISTRIBUTION BOX/PANEL).

****************************************

NOW THAT WE HAVE THE BASICS, I'LL TRY

AND GO IN-DEPTH ON THE SUBJECT.

------------------

UNDERGROUND CABLES

------------------

THESE ARE SOMETIMES INTER-

OFFICE TRUNKS, BUT USUALLY IN A RESID-

ENTIAL AREA THEY ARE FEEDER GROUPS THAT

GO OTO BRIDGING HEADS OR DISTRIBUTION

CASES. THE CABLES ARE ABOUT 2-3 INCHES

THICK (VARIES), AND ARE EITHER IN A

METAL OR PVC-TYPE PIPE (OR SIMILAR).

RARELY (MAYBE NOT IN SOME REMOTE RURAL

AREAS) ARE THE CABLES JUST 'ALONE' IN

THE GROUND. INSTEAD, THEY ARE USUALLY

IN AN UNDERGROUND CEMENT TUNNEL

(RESEMBLES A SMALL SEWER OR STORMDRAIN)

. THE MANHOLES ARE >HEAVY< AND WILL SAY

'BELL SYSTEM' ON THEM. THEY CAN BE OPEN

ED WITH A 1/2 INCH WIDE CROWBAR (HOOK

SIDE) INSERTED IN THE TOP RECTANGULAR

HOLE. IF YOU GET IT OPEN, GO INSIDE!!

THERE ARE LADDER RUNGS TO HELP YOU

CLIMB DOWN. YOU WILL SEE THE CABLE

PIPES ON THE WALL, WITH THE BLUE AND

WHITE STRIPED ONE BEING THE INTER-

OFFICE TRUNK GRP (AT LEAST IN MY AREA).

THE OTHERS ARE LOCAL LINES, AND ARE

USUALLY MARKED OR COLOR CODED. THERE

IS ALMOST ALWAYS A POSTED COLOR CODE

CHART ON THE WALL, NOT TO MENTION

TELCO MANUALS DESCRIBING THE CABLES

AND TERMINALS, SO I NEED NOT GET INTO

DETAIL. AGAIN: >IF YOU CAN GET INTO A

BELL MANHOLE, DO IT!, IT WILL PAY OFF

<. ALSO, THERE IS USUALLY SOME KIND

OF TEST EQUIPMENT, AND OFTEN BELL

TEST SETS ARE LEFT IN THERE.

SO GET YOUR CROWBARS!

****************************************

--------------

BRIDGING HEADS - (WE MS2'S)

--------------

THE INNOCENT-LOOKING GRAYISH-

GREEN BOXES.

WHEN IN GROUPS OF TWO OR THREE, THEY

ARE FOR SECONDAY/TERTIARY TERMINATION

AND ACCESS POINTS (BRIDGED ACCESS).

THESE ARE THE WESTERN ELECTRIC SAI

55 OR 22/E TERMINAL CASES. THEY HOLD

ON THE ORDER OF 900-1200 CABLE PAIRS.

THIS IS USUALLY THE SECOND TERM. POSI

TION ON THE LOCAL LOOP (AFTER THE MTF

FRAME AT THE CO/ OR THE UNDERGROUND

'FTP' POSITION).

THESE CAN BE EITHER

TRUNK BRIDGES OR BRIDGING FOR RESIDEN

CES. THE AREA FEEDER BRIDGING HEADS

ARE USUALLY LARGER, AND THEY HAVE THE

'WESTERN ELECTRIC' LOGO AT THE BOTTOM,

WHEREAS THE NORMAL BRIDGING HEADS

(WHICH MAY BE DIFFERENT IN SOME AREAS-

DEPENDING ON THE COMPANY YOU ARE

SERVED BY. GTE B.H'S LOOK SLIGHTLY

DIFFERENT. ALSO, DO NOT BE FOOLED

BY SPRINKLER BOXES!)

CAN BE FOUND IN JUST ABOUT EVERY CITY.

TO OPEN A BRIDGING HEAD:

IF IT IS LOCKED (AND YOU'RE FEELING

DESTRUCTIVE), PUT A HAMMER OR CROWBAR

(THE SAME ONE YOU USED ON THE MANHOLE

) IN THE SLOT ABOVE THE TOP HINGE OF

THE RIGHT DOOR. PULL HARD, AND THE

DORR WILL RIP OFF. VERY EFFECTIVE!

IF IT ISN'T LOCKED (AS USUAL),

TAKE A 7/16 INCH HEX SOCKET AND

WITH IT, TURN THE BOLT ABOUT 1/8 OF A

TURN TO THE LEFT (YOU SHOULD HEAR

A SPRING RELEASE INSIDE). HOLDING THE

BOLT, TURN THE HANDLE ALL THE WAY TO

THE RIGHT AND PULL OUT.

NOW INSIDE, FIRST CHECK FOR

A TEST-SET (WHICH ARE OFTEN LEFT BY

BELL EMPLOYEES). THERE SHOULD BE A

PANEL OF CABLE PAIRS + SCREW TERMINALS

(TYPE 45/47IB REGULAR BINDERS) . PUSH

THE PANEL BACK ABOUT AN INCH OR SO,

AND ROTATE THE TOP LATCH (ROUND WITH

A FLAT SECTION) DOWNWARD. RELEASE THE

PANEL AND IT WILL FALL ALL THE WAY

FORWARD. THERE IS USUALLY A LARGE AMO-

UNT OF WIRE AND EXTRA TERMINALS.

THE TEST-SETS ARE OFTEN HIDDEN HERE,

SO DONT OVERLOOK IT (400 FOOT ROLLS OF

#22 SOLID WIRE ARE OFTEN NEAR THE TOP

IN THE BACK OF THE BOX. 'BORROW THEM')

. ON THE RIGHT DOOR IS A METAL BOX OF

INSULATORS, BINDERS, CLIPS, ETC.  TAKE

A FEW (COMPLIMENTS OF BELL...). ON EACH DOOR IS A USEFUL ROUND METAL DEVICE.

(SAYS 'INSERT GENTLY' OR 'CLAMP GENTLY

- DO NOT OVERTIGHTEN' ETC..) ON THE

FRONT OF THE DISC, YOU SHOULD FIND TWO

TERMINALS. THESE ARE FOR YOUR TEST SET.

(IF YOU DONT HAVE ONE, DONT DEPAIR -

I'LL SHOW YOU WAYS TO MAKE BASIC TEST

SETS LATER IN THIS ARTICLE).

HOOKING THE RING (-) WIRE TO THE

'R' TERMINAL; AND THE TIP (+) WIRE

TO THE OTHER. (BY THE WAY, AN EASY WAY

TO DETERMINE THE CORRECT POLARITY IS

WITH A 1.5V LED. TAP IT TO THE TERM.

PAIR, IF I DOESNT LIGHT, SWITCH THE

POLES UNTIL IT DOES. WHEN IT LIGHTS,

FIND THE LONGER OF THE TWO LED POLES.

THIS ONE WILL BE ON THE TIP WIRE (+))

BEHIND THE DISC IS A COILED UP CORD.

THIS SHOULD HAVE A SPECIALIZED CONNECT-

OR. ITS VERY USEFUL, BECAUSE YOU

DONT HAVE TO KEEP CONNECTING AND

DISCONNECTING THE FONE (TEST SET)

ITSELF, AND THE CLIP IS DESIGNED TO AID

IN LOCATING THE PAIR AND ASSURING THE

CORRECT POLARITY.

ON THE TERMINAL BOARD, THERE

SHOULD BE ABOUT 20 PAIRS (RED/WHITE)

PER ROW PER SIDE.

HOOK THE CLIP TO ANY TERMINAL PAIR

, AND YOU'RE SET! DIAL OUT IF YOU WANT,

OR JUST LISTEN (IF SOMEONE'S ON THE

LINE). LATER, I'LL SHOW YOU A WAY TO

SET UP A TRUE 'TAP' THAT YOU CAN SET

UP, AND WILL LET THE PERSON DIAL OUT

ON HIS LINE AND RECEIVE CALLS AS

NORMAL, AND YOU CAN LISTEN IN THE

WHOLE TIME. MORE ABOUT THIS LATER...

ON MAJOR PREFIX-AREA BRIDGING

HEADS ('SAI' FOR AREA C.O. FEEDER CABLE

) YOU CAN SEE TEMPORARY 'LOCAL LOOPS',

WHICH ARE TWO CABLE PAIRS (CABLE

PAIR = RING+TIP, A FONE LINE) THAT

ARE DIRECTLY CONNECTED TO EACH OTHER

ON THE TERMINAL BOARD. THESE 'CHEAP

LOOPS' AS THEY ARE CALLED, DO NOT

WORK NEARLY AS WELL AS THE EXISTING

ONES SET UP IN THE SWITCHING HARDWARE

AT THE EXCHANGE OFFICE. (TRY SCANNING

YOUR PREFIXES' 00XX OR 99XX #'S.

THE TONE SIDES WILL ANNOUNCE THEMSELVES

WITH THE 1000 HZ LOOP TONE, AND THE

HANG SIDE WILL GIVE NO RESPONSE. THE

FIRST PERSON SHOULD DIAL THE 'HANG'

SIDE, AND THE OTHER PERSON DIAL THE,

TONE SIDE, AND THE TONE SHOULD STOP

IF YOU HAVE GOT THE RIGHT LOOP).

THE LOOPS ARE USED IN FRAME TERMINATI

ON AND IN SONIC/600 OHM TERM. TESTING

AND LINE/CABLE/FEEDER LOCATION.

IF YOU WANT TO FIND THE NUMBER

OF THE LINE THAT YOU'RE ON, YOU CAN

EITHER TRY TO DECIPHER THE 'BRIDGING

LOG' (OR WHATEVER), WHICH IS ON THE

LEFT DOOR. IF THAT DOESNT WORK, YOU

CAN USE THE FOLLWING:

---

ANI # (AUTOMATIC NUMBER INDENTICATION)

---

THIS IS A TELCO TEST NUMBER

THAT REPORTS TO YOU THE NUMBER THAT

YOURE CALLING FROM (IT'S THE SAME,

CHOPPY 'BELL b****!' VOICE THAT YOU

GET WHEN YOU REACH A DISCONNECTED #)

(PIP OR ONE OF THE 11X SERIES ANNOUNCE

MENT FRAME MACHINES.)

FOR THE 213 NPA - DIAL 1223

213 NPA - (GTE AREAS) DIAL 114

408 NPA - DIAL 760

914 NPA - DIAL 990

THESE ARE EXTREMELY USEFUL WHEN MESSING

WITH ANY KIND OF LINE TERMINALS,

FEEDER BOXES, ETC.

----------

WHEN FINISHED, BE SURE TO CUT

OFF THE CONNECTOR AND TAKE IT WITH YOU

(THEY ARE VERY HANDY), AND CLOSE/LATCH

THE BOX TO AVOID SUSPICION.

***************************************

------

"CANS" - AERIAL-MOUNTED (TELEPHONE POLE

------   TERMINAL) BRANCH FEEDER CABLE

SPLITTER.

BASICALLY, TWO TYPES:

1> MS3A SERVICE AREA INTERFACE.

(LARGE, RECTANGULAR SILVER BOX AT

THE END OF EACH STREET.)

2> SERIES 600 LOADING COIL, INCLUDES

THE MSX AERIAL DROP SPLITTER.

(BLACK, ROUND OR RECTANGULAR THING

AT EVERY TELEPHONE POLE.)

--------

TYPE 1 -   THIS IS THE CASE THAT TAKES

THE UNDERGROUND CABLE FROM THE BRIDGER

AND RUNS IT TO THE AERIAL BRANCH FEEDER

CABLE (THE LOWEST, LARGEST WIRE ON THE

TELEPHONE POLE). THE BOX IS ALWAYS ON

THE POLE NEAREST THE BRIGING HEAD,

WHERE THE LINE COMES UP. LOOK FOR THE

'CALL BEFORE YOU DIG - UNDERGROUND

CABLE' STICKERS..

THE CASE BOX IS HINGED, SO IF

YOU WANT TO CLIMB THE POLE, YOU CAN

OPEN IT WITH NO PROBLEMS. THESE USUALLY

HAVE 2 ROWS OF TERMINAL SETS.

(TOTAL APPROX. 200-600 PAIRS, DEPENDING

ON THE AREA.) THESE ARE ALL THE CABLE

PAIRS FOR YOUR STREET.

. (ITS SIMILAR TO A MINIATURE BRIDGING

HEAD). USE/ABUSE IT IN THE SAME MANNER

AS WE DID BEFORE. (NOTE: ALL THE ACTIVE

LINES CARRY FROM 15 TO 48 VDC, AND

EVEN 90VAC (WHEN RINGING), SO BE

CAREFUL - IT'S NOT GOING TO HURT YOU,

BUT IT CAN SURPRISE YOU (AND IF

YOU'RE HANGING BY ONE HAND FROM A TEL.

POLE, IT >CAN< BE HARMFUL!))

OH, BY THE WAY, IF YOU USE

ANI ON EVERY PAIR AND YOU FIND ONE

THAT ISNT IN USE ON YOUR STREET, YOU

CAN HOOK IT UP FOR YOURSELF (ALMOST).

ALSO, YOU HAVE TO BE ABLE TO IMPER-

SONATE A TELCO TECHNICIAN AND REPORT

THE NUMBER AS 'NEW ACTIVE' (GIVING

A FAKE NAME AND FAKE REPORT, ETC)

I DONT RECOMMEND THIS AND IT PROBABLY

WONT (ALMOST POSITIVELY WONT) WORK,

BUT THIS IS BASICALLY WHAT TELCO

LINEMEN DO).*

* THE USE OF THE 'TEST BOARD'

(A 7D# FOR LINEMEN TESTS, OFTEN FOUND

NXX-0003 OR NXX-0004) IS BEYOND THE

SCOPE OF THIS ARTICLE, BUT WILL BE

COVERED IN A FUTURE ARTICLE.

----

TYPE 2 - THIS IS THE SPLITTER BOX FOR

THE AERIAL DROP WIRES FOR THE GROUP OF

HOUSES AROUND THE POLE.

(USUALLY 4 OR 5 HOUSES). USE IT

LIKE I MENTIONED BEFORE. THE TERMINALS

(20 OR SO) WILL BE IN 2 HORIZONTAL

ROWS OF SETS. THE EXTRA WIRES THAT ARE

JUST 'HANGING THERE' ARE PROVISIONS

FOR EXTRA LINES TO RESIDENCES (1 EXTRA

LINE PER HOUSE, THATS WHY THE INSANE

CHARGE FOR LINE #3). IF ITS THE BOX FOR

YOUR HOUSE ALSO, HAVE FUN AND SWAP

LINES WITH YOUR NEIGHBOR! 'PIGGYBACK'

THEM OR USE A CHEESEBOX FOR A LOOP-ARO

UND, ETC.

AGAIN, I DONT RECOMMEND THIS, AND ITS

DIFFICULT TO DO IT CORRECTLY. (ALTHOUGH

USEFUL)

***************************************

---------------------------------------

DOWNLOADED FROM P-80 SYSTEMS.