/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/

/-/                           /-/

/-/        PHREAKER'S         /-/

/-/         PHUNHOUSE         /-/

/-/                           /-/

/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/

/-/   BY:                     /-/

/-/       THE TRAVELER        /-/

/-/                           /-/

/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/-/

THE LONG AWAITED PREQUIL TO PHREAKER'S GUIDE HAS FINALLY ARRIVED.

CONCEIVED FROM THE BOREDOM AND LONELINESS THAT COULD ONLY BE DERIVED FROM:

THE TRAVELER!  BUT NOW, HE HAS RETURNED IN FULL STRENGTH (AFTER A SMALL

VACATION) AND IS HERE TO 'WORLD PREMIERE' THE NEW FILES EVERYWHERE. STAY

COOL. THIS IS THE PREQUIL TO THE FIRST ONE, SO JUST RELAX. THIS IS NOT MADE

TO BE AN EXCLUSIVE ULTRA ELITE FILE, SO KINDA CALM DOWN AND WATCH IN THE

BACKGROUND IF YOU ARE TOO COOL FOR IT.

/-/   PHREAK DICTIONARY   /-/

HERE YOU WILL FIND SOME OF THE BASIC BUT NECESSARY TERMS THAT SHOULD BE

KNOWN BY ANY PHREAK WHO WANTS TO BE RESPECTED AT ALL.

PHREAK  : 1. THE ACTION OF USING MISCHEVIOUS AND MOSTLY ILLEGAL

WAYS IN ORDER TO NOT PAY FOR SOME SORT OF TELE-

COMMUNICATIONS BILL, ORDER, TRANSFER, OR OTHER SERVICE.

IT OFTEN INVOLVES USAGE OF HIGHLY ILLEGAL BOXES AND

MACHINES IN ORDER TO DEFEAT THE SECURITY THAT IS SET

UP TO AVOID THIS SORT OF HAPPENING. [FR'EAKING]. V.

2. A PERSON WHO USES THE ABOVE METHODS OF DESTRUCTION AND

CHAOS IN ORDER TO MAKE A BETTER LIFE FOR ALL.  A TRUE

PHREAKER WILL NOT NOT GO AGAINST HIS FELLOWS OR NARC

ON PEOPLE WHO HAVE RAGGED ON HIM OR DO ANYTHING

TERMED TO BE DISHONORABLE TO PHREAKS. [FR'EEK]. N.

3. A CERTAIN CODE OR DIALUP USEFUL IN THE ACTION OF

BEING A PHREAK. (EXAMPLE: "I HACKED A NEW METRO

PHREAK LAST NIGHT.")

SWITCHING SYSTEM: 1. THERE ARE 3 MAIN SWITCHING SYSTEMS CURRENTLY EMPLOYED

IN THE US, AND A FEW OTHER SYSTEMS WILL BE MENTIONED

AS BACKGROUND.

A) SXS: THIS SYSTEM WAS INVENTED IN 1918 AND WAS

EMPLOYED IN OVER HALF OF THE COUNTRY UNTIL 1978. IT

IS A VERY BASIC SYSTEM THAT IS A GENERAL WASTE OF

ENERGY AND HARD WORK ON THE LINESMAN. A GOOD WAY TO

IDENTIFY THIS IS THAT IT REQUIRES A COIN IN THE PHONE

BOOTH BEFORE IT WILL GIVE YOU A DIAL TONE, OR THAT NO

CALL WAITING, CALL FORWARDING, OR ANY OTHER SUCH

SERVICE IS AVAILABLE.  STANDS FOR: STEP BY STEP

B) XB: THIS SWITCHING SYSTEM WAS FIRST EMPLOYED IN 1978

IN ORDER TO TAKE CARE OF MOST OF THE FAULTS OF SXS

SWITCHING.  NOT ONLY IS IT MORE EFFICIENT, BUT IT

ALSO CAN SUPPORT DIFFERENT SERVICES IN VARIOUS FORMS.

XB1 IS CROSSBAR VERSION 1. THAT IS VERY LIMITED AND

IS HARD TO DISTINGUISH FROM SXS EXCEPT BY DIRECT VIEW

OF THE WIRING INVOLVED. NEXT UP WAS XB4, CROSSBAR

VERSION 4. WITH THIS SYSTEM, SOME OF THE BASIC THINGS

LIKE DTMF THAT WERE NOT AVAILABLE WITH SXS CAN BE

ACCOMPLISHED. FOR THE FINAL STROKE OF XB, XB5 WAS

CREATED. THIS IS A SERVICE THAT CAN ALLOW DTMF PLUS

MOST 800 TYPE SERVICES (WHICH WERE NOT ALWAYS

AVAILABLE.) STANDS FOR: CROSSBAR.

C) ESS: A NIGHTMARE IN TELECOM. IN VIVID COLOR, ESS IS

A PRETTY BAD THING TO HAVE TO STAND UP TO. IT IS

QUITE SIMPLE TO IDENTIFY. DIALING 911 FOR EMERGENCIES,

AND ANI [SEE ANI BELOW] ARE THE MOST COMMON FACETS OF

THE DREAD SYSTEM. ESS HAS THE CAPABILITY TO LIST IN A

PERSON'S CALLER LOG WHAT NUMBER WAS CALLED, HOW LONG

THE CALL TOOK, AND EVEN THE STATUS OF THE CONVERSATION

(MODEM OR OTHERWISE.) SINCE ESS HAS BEEN EMPLOYED,

WHICH HAS BEEN VERY RECENTLY, IT HAS GONE THROUGH

MANY KINDS OF REVISIONS. THE LATEST SYSTEM TO DATE IS

ESS 11A, THAT IS EMPLOYED IN WASHINGTON D.C. FOR

SECURITY REASONS. ESS IS TRULY TROUBLE FOR ANY

PHREAK, BECAUSE IT IS 'SMARTER' THAN THE OTHER

SYSTEMS. FOR INSTANCE, IF ON YOUR CALLER LOG THEY SAW

50 CALLS TO 1-800-421-9438, THEY WOULD BE ABLE TO DO

A CN/A [SEE LOOPHOLES BELOW] ON YOUR NUMBER AND

DETERMINE WHETHER YOU ARE SUBSCRIBED TO THAT SERVICE

OR NOT. THIS MAKES MOST CALLS A HAZARD, BECAUSE

ALTHOUGH 800 NUMBERS APPEAR TO BE FREE, THEY ARE

RECORDED ON YOUR CALLER LOG AND THEN RIGHT BEFORE YOU

RECEIVE YOUR BILL IT DELETES THE BILLINGS FOR THEM.

BUT BEFORE THAT THE ARE OPEN TO INSPECTION, WHICH IS

ONE REASON WHY EXTENDED USE OF ANY CODE IS DANGEROUS

UNDER ESS. SOME OF THE BOXES [SEE BOXING BELOW] ARE

UNABLE TO FUNCTION IN ESS.  IT IS GENERALLY A MENACE

TO THE TRUE PHREAK. STANDS FOR: ELECTRONIC SWITCHING

SYSTEM. BECAUSE THEY COULD APPEAR ON A FILTER

SOMEWHERE OR MAYBE IT IS JUST NICE TO KNOW THEM

ANYWAYS.

A) SSS: STROWGER SWITCHING SYSTEM. FIRST

NON-OPERATOR SYSTEM AVAILABLE.

B) WES: WESTERN ELECTRONICS SWITCHING. USED ABOUT 40

YEARS AGO WITH SOME MINOR PLACES OUT WEST.

BOXING:  1) THE USE OF PERSONALLY DESIGNED BOXES THAT EMIT OR

CANCEL ELECTRONICAL IMPULSES THAT ALLOW SIMPLER

ACTING WHILE PHREAKING. THROUGH THE USE OF SEPARATE

BOXES, YOU CAN ACCOMPLISH MOST FEATS POSSIBLE WITH

OR WITHOUT THE CONTROL OF AN OPERATOR.

2) SOME BOXES AND THEIR FUNCTIONS ARE LISTED BELOW.

ONES MARKED WITH '*' INDICATE THAT THEY ARE NOT

OPERATABLE IN ESS.

*BLACK BOX: MAKES IT SEEM TO THE PHONE COMPANY THAT

THE PHONE WAS NEVER PICKED UP.

BLUE BOX  : EMITS A 2600HZ TONE THAT ALLOWS YOU TO DO

SUCH THINGS AS STACK A TRUNK LINE, KICK

THE OPERATOR OFF LINE, AND OTHERS.

RED BOX : SIMULATES THE NOISE OF A QUARTER, NICKEL,

OR DIME BEING DROPPED INTO A PAYPHONE.

CHEESE BOX : TURNS YOUR HOME PHONE INTO A PAY PHONE TO

THROW OFF TRACES (A RED BOX IS USUALLY

NEEDED IN ORDER TO CALL OUT.)

*CLEAR BOX : GIVES YOU A DIAL TONE ON SOME OF THE OLD

SXS PAYPHONES WITHOUT PUTTING IN A COIN.

BEIGE BOX : A SIMPLER PRODUCED LINESMAN'S HANDSET THAT

ALLOWS YOU TO TAP INTO PHONE LINES AND

EXTRACT BY EAVESDROPPING, OR CROSSING

WIRES, ETC.

PURPLE BOX : MAKES ALL CALLS MADE OUT FROM YOUR HOUSE

SEEM TO BE LOCAL CALLS.

ANI [ANI]: 1) AUTOMATIC NUMBER IDENTIFICATION. A SERVICE

AVAILABLE ON ESS THAT ALLOWS A PHONE SERVICE [SEE

DIALUPS BELOW] TO RECORD THE NUMBER THAT ANY CERTAIN

CODE WAS DIALED FROM ALONG WITH THE NUMBER THAT WAS

CALLED AND PRINT BOTH OF THESE ON THE CUSTOMER BILL.

950 DIALUPS [SEE DIALUPS BELOW] ARE ALL DESIGNED

JUST TO USE ANI. SOME OF THE SERVICES DO NOT HAVE

THE PROPER EQUIPMENT TO READ THE ANI IMPULSES YET,

BUT IT IS IMPOSSIBLE TO SEE WHICH IS WHICH WITHOUT

BEING BUSTED OR NOT BUSTED FIRST.

DIALUPS [DY'L'UPS]: 1) ANY LOCAL OR 800 EXTENDED OUTLET THAT ALLOWS INSTANT

ACCESS TO ANY SERVICE SUCH AS MCI, SPRINT, OR AT&T

THAT FROM THERE CAN BE USED BY HANDPICKING OR USING

A PROGRAM TO REVEAL OTHER PEOPLES CODES WHICH CAN

THEN BE USED MODERATELY UNTIL THEY FIND OUT ABOUT

IT AND YOU MUST SWITCH TO ANOTHER CODE (PREFERRABLY

BEFORE THEY FIND OUT ABOUT IT.)

2) DIALUPS ARE EXTREMELY COMMON ON BOTH SENSES. SOME

DIALUPS REVEAL THE COMPANY THAT OPERATES THEM AS

SOON AS YOU HEAR THE TONE. OTHERS ARE MUCH HARDER

AND SOME YOU MAY NEVER BE ABLE TO IDENTIFY.  A SMALL

LIST OF DIALUPS:

3) CODES: CODES ARE VERY EASILY ACCESSED PROCEDURES

WHEN YOU CALL A DIALUP. THEY WILL GIVE YOU SOME SORT

OF TONE.  IF THE TONE DOES NOT END IN 3 SECONDS,

THEN PUNCH IN THE CODE AND IMMEDIATELY FOLLOWING THE

CODE, THE NUMBER YOU ARE DIALING BUT STRIKE THE

'1' IN THE BEGINNING OUT FIRST. IF THE TONE DOES

END, THEN PUNCH IN THE CODE WHEN THE TONE ENDS.

THEN, IT WILL GIVE YOU ANOTHER TONE.  PUNCH IN THE

NUMBER YOU ARE DIALING, OR A '9'. IF YOU PUNCH IN

A '9' AND THE TONE STOPS, THEN YOU MESSED UP A

LITTLE. IF YOU PUNCH IN A TONE AND THE TONE

CONTINUES, THEN SIMPLY DIAL THEN NUMBER YOU ARE

CALLING WITHOUT THE  '1'.

4) ALL CODES ARE NOT UNIVERSAL. THE ONLY TYPE THAT I

KNOW OF THAT IS TRULY UNIVERSAL IS METROPHONE.

ALMOST EVERY MAJOR CITY HAS A LOCAL METRO DIALUP

(FOR PHILADELPHIA, (215)351-0100/0126) AND SINCE THE

CODES ARE UNIVERSAL, ALMOST EVERY PHREAK HAS USED

THEM ONCE OR TWICE. THEY DO NOT EMPLOY ANI IN ANY

OUTLETS THAT I KNOW OF, SO FEEL FREE TO CHECK

THROUGH YOUR BOOKS AND CALL 555-1212 OR, AS A MORE

DEVIOUS MANOR, SUBSCRIBE YOURSELF. THEN, NEVER USE

YOUR OWN CODE. THAT WAY, IF THEY CHECK UP ON YOU DUE

TO YOUR CALLER LOG, THEY CAN USUALLY FIND OUT THAT

YOU ARE SUBSCRIBED.  NOT ONLY THAT BUT YOU COULD SET

A PHREAK HACKER AROUND THAT AREA AND JUST LET IT

HACK AWAY, SINCE THEY USUALLY GROUP THEM, AND, AS A

BONUS, YOU WILL HAVE THEIR LOCAL DIALUP.

5) 950'S. THEY SEEM LIKE A PERFECTLY COOL PHREAKERS

DREAM. THEY ARE FREE FROM YOUR HOUSE, FROM PAYPHONES,

FROM EVERYWHERE, AND THEY HOST ALL OF THE MAJOR LONG

DISTANCE COMPANIES (950-1044 <MCI>, 950-1077

<SPRINT>, 950-1088 <SKYLINES>, 950-1033 <US

TELECOM>.) WELL, THEY AREN'T. THEY WERE DESIGNED FOR

ANI. THAT IS THE POINT, END OF DISCUSSION.

PHREAKER'S DICTIONARY

^^^^^^^^^^^^^^^^^^^^^

A & A BUREAU--ABUSE AND ANNOYANCE BUREAU. THE PERSONNEL IN THIS

LINE OF WORK SPEND THEIR TIME HELPING CUSTOMERS GET RID OF NUTS,

OBSCENE CALLERS, HARASSING COLLECTORS, ETC.

ACCESS--THE EXISTENCE OF PATHS WITHIN A NETWORK FROM AN INPUT

TERMINAL TO A SET OF OUTPUT TERMINALS IN THE ABSENCE OF TRAFFIC

IS INDICATED BY THE TERM, ACCESS. FULL ACCESS PERMITS CONNECTING

TO ALL OUTPUT TERMINALS BY UNIQUE PATHS; MULTIPLE ACCESS

INDICATES THAT ALL OUTPUT TERMINALS CAN BE REACHED IN MORE THAN

ONE WAY; PARTIAL ACCESS REFERS TO THE ABILITY TO REACH ONLY A

FRACTION OF THE OUTPUT TERMINALS.

ACCESSIBILITY--(AVAILABILITY)--THE NUMBER OF TRUNKS OF THE

REQUIRED ROUTE IN A SWITCHING NETWORK WHICH CAN BE REACHED FROM

AN INLET.

ADAPTOR--A DEVICE DESIGNED TO SWITCH A NUMBER OF VOICE-FREQUENCY

TELEPHONE CHANNELS COMING FROM A NON-TIME-DIVISION SWITCHING

SYSTEM TO A TIME-DIVISION MULTIPLEX HIGHWAY.

ALTERNATE ROUTING--A PROCEDURE BY WHICH SEVERAL ROUTES INVOLVE

DIFFERENT SWITCHING STAGES OR SWITCHING NETWORKS. USUALLY THE

ROUTE HAVING THE FEWEST SWITCHING STAGES IS TESTED FIRST.

ANALOG TRANSMISSION--THE TRANSMISSION OF CONTINUOUSLY VARIABLE

SIGNALS RATHER THAN DESCRETELY VARIABLE SIGNALS. PRIOR TO THE USE

OF DIGITAL ENCODING AND PCM, IT WAS THE ONLY WAY OF TRANSMITTING

VOICE SIGNALS OVER TELEPHONE CHANNELS.

AREA CODE--A THREE-DIGIT PREFIX DIALED AHEAD OF THE NORMAL

SEVEN-DIGIT TELEPHONE NUMBER TO PERMIT DIRECT DISTANCE DIALING.

ASYNCHRONOUS SYSTEM--A SYSTEM IN WHICH THE TRANSMISSION OF EACH

INFORMATION CHARACTER IS INDIVIDUALLY SYNCHRONIZED USUALLY BY THE

USE OF START AND STOP ELEMENTS.

AVERAGE HOLDING TIME--THE AVERAGE DURATION OF A CALL EXPRESSED IN

SECONDS OR MINUTES.

BIT--THE SMALLEST BINARY UNIT OF INFORMATION. A CONTRACTION OF

THE WORDS BINARY DIGIT.

BIT RATE--THE RATE OR SPEED AT WHICH BITS ARE TRANSMITTED. BITS

PER SECOND IS A COMMON MEASURE.

BLOCKING (CONGESTION)--A CONDITION WHERE THE IMMEDIATE

ESTABLISHMENT OF A NEW CONNECTION IS IMPOSSIBLE DUE TO THE LACK

OF AVAILABLE PATHS, OR THE INABILITY TO INTERCONNECT TWO IDLE

NETWORK TERMINALS BECAUSE SOME OF THE APPLICABLE LINKS BETWEEN

THEM ARE USED FOR OTHER CONNECTIONS.

BOOLEAN ALGEBRA--A FORM OF NONQUANTITATIVE ALGEBRA FOR DEALING

WITH LOGIC FUNTIONS, ORIGINALLY EXPRESSED BY BRITISH

MATHEMATICIAN GEORGE BOOLE (1815-1864).

B.O.S.--BUSINESS OFFICE SUPERVISOR. SHE'S THE BOSS TO THE SERVICE

REPS.

BROADBAND EXCHANGE (BEX)--PUBLIC SWITCHED COMMUNICATION SYSTEM

FEATURING FULL DUPLEX (FDX) CONNECTIONS OF VARIOUS BANDWIDTHS. A

WESTERN UNION FACILITY.

B.S.I.--BUSINESS SERVICES INSTRUCTOR. A TRAFFICE EMPLOYEE WHO

WILL COME OUT AND TEACH YOU HOW TO USE YOUR PHONE SYSTEM.

BUSY HOUR--AN UNINTERRUPTED PERIOD OF 60 MINUTES IN WHICH THE

TOTAL TRAFFIC OF A SAMPLE IS A MAXIMUM.

BYTE--A UNIT OF INFORMATION IN ELECTRONIC COMPUTER TERMINOLOGY

CONSISTING OF 8 BITS, REFERRED TO AS EXTENDED BINARY CODED

DECIMAL INFORMATION OF AN EBCDIC CODE.

CALL CONGESTION RATIO--THE RATIO OF THE TIME DURING WHICH

CONGESTION EXISTS TO THE TOTAL TIME CONSIDERED. IT IS AN ESTIMATE

OF THE PROBABILITY THAT AN EXTERNAL OBSERVER WILL FIND A SYSTEM

IN A CONGESTED CONDITION.

CALLING RATE--AVERAGE CALLS PER SUBSCRIBER PER HOUR.

CALL STORE--THE MEMORY SECTION OF A STORED PROGRAM CONTROL

SWITCHING SYSTEM IN WHICH TEMPORARY INFORMATION USED IN THE

PROCESSING OF CALLS THROUGH THE EXCHANGE IS CONTAINED. IT IS ALSO

REFERRED TO AS THE PROCESS STORE.

CENTRAL OFFICE--EXCHANGES WHERE SUBSCRIBER LINES AND PRIVATE

BRANCH EXCHANGE LINES TERMINATE. THERE THEY ARE SWITCHED TO

PROVIDE THE DESIRED CONNECTION WITH OTHER SUBSCRIBERS. SUCH AN

EXCHANGE IS CALLED AN END OFFICE AND IS DESIGNATED AS A CLASS 5

OFFICE IN THE U.S.

CENTRAL PROCESSOR--THE MAIN COMPUTER ELEMENT OF A STORED PROGRAM

CONTROL SWITCHING SYSTEM, WHICH UNDER THE DIRECTION OF THE STORED

PROGRAM ESTABLISHES SWITCHING NETWORK CONNECTIONS AND ALSO

MONITORS AND ANALYZES THE SYSTEM TO INSURE PROPER OPERATION.

ROUTINE PROCESS TESTING, MAINTENANCE AND ADMINISTRATIVE FUNTIONS

ARE ALSO CARRIED OUT.

CENTREX--A PABX SYSTEM IN WHICH THE SWITCHING EQUIPMENT IS

LOCATED CENTRALLY AND AWAY FORM THE LOCATION BEING SERVED. DIRECT

INWARD DIALING (DID) AND DIRECT OUTWARD DIALING (DOD) AS WELL AS

AUTOMATIC NUMBER IDENTIFICATION (ANI) ARE PROVIDED BY SUCH A

SYSTEM.

CHARACTERS--THE ELEMENTS OF A MESSAGE. ONE COMPUTER CHARACTER

CONSISTS OF 8 BITS OR 1 BYTE AND IS KNOWN AS AN EBCDIC CHARACTER.

CIRCUIT SWITCHING--TELECOMMUNICATIONS SWITCHING IN WHICH THE

INCOMING AND OUTGOING LINES ARE CONNECTED BY A PHYSICAL PATH, AS

THROUGH CROSSPOINTS OR SWITCH CONTACTS.

CLASS OF SERVICE--THE SERVICES AND FACILITIES OFFERED TO EACH

INDIVIDUAL TERMINAL CONNECTED TO A SYSTEM. THIS INFORMATION IS

USUALLY STORED WITH THE DIRECTORY OR EQUIPMENT NUMBERS OF THE

ASSOCIATED TERMINAL, AND IS ACCESSED BY THE CALL PROCESSORS WHEN

A CONNECTION IS REQUIRED TO OR FROM THAT TERMINAL.

CLOCK--EQUIPMENT TO PROVIDE A TIME BASE FOR A SWITCHING SYSTEM.

IN TIME-DIVISION SWITCHING IT IS USED TO CONTROL SAMPLING RATES,

DURATION OF SIGNAL DIGITS, ETC.

C.O.A.M.E.--CUSTOMER OWNED AND MAINTAINED EQUIPMENT.

CODEC--THE COMBINATION OF A CODER AND DECODER, AS USED IN

TIME-DIVISION SWITCHING SYSTEMS TO CODE THE INCOMING MESSAGE AND

DECODE THE MESSAGE BEING RETURNED TO THE CALLER. IT IS A

CONTRACTION OF THE WORDS, CODER AND DECODER.

COMMON CONTROL--AN EXCHANGE CONTROL METHOD IN WHICH THE DIALED

SIGNALS ARE RECEIVED AND REGISTERED SEPARATELY FROM THE SWITCHING

ELEMENTS BEFORE THEY ARE USED TO CONTROL THESE SWITCHES. ALSO

DEFINED AS A CONTROL METHOD, WHICH IDENTIFIES THE INPUT AND

OUTPUT TERMINALS OF THE SWITCHING NETWORK AND THEN CAUSES A

CONNECTING PATH TO BE ESTABLISHED BETWEEN THEM. SUCH SYSTEMS ARE

ALSO DESIGNATED AS MARKER SYSTEMS.

CONCENTRATION STAGE--A SWITCHING STAGE IN WHICH A NUMBER OF INPUT

LINES ARE CONNECTED TO A SMALLER NUMBER OF OUTPUT LINES OR

TRUNKS, AS IN THE CONNECTION OF A LARGE NUMBER OF SUBSCRIBER

LINES TO A SMALLER NUMBER OF TRUNKS BASED ON THE GRADE OF SERVICE

DESIRED.

CONGESTION FUNCTION--ANY FUNCTION USED TO RELATE THE DEGREE OF

CONGESTION TO THE TRAFFIC INTENSITY.

CONNECTING ROW--ALL THOSE CROSSPOINTS DIRECTLY ACCESSIBLE FROM AN

INLET. ONLY ONE CONNECTION CAN BE ESTABLISHED VIA A CONNECTING

ROW AT ANY INSTANT.

COUPLER--A DEVICE USED TO PREVENT ELECTRICAL FLASHBACK AND

MAINTAIN NORMAL ELECTRICAL FLOW ON A TELEPHONE LINE. USED AS A

BUFFER BETWEEN C.O.A.M.E. AND TELEPHONE COMPANY EQUIPMENT.

CROSSBAR SWITCH--A SWITCH HAVING A PLURALITY OF VERTICAL PATHS, A

PLURALITY OF HORIZONTAL PATHS AND ELECTROMAGNETICALLY OPERATED

MECHANICAL MEANS FOR CONNECTING ANY OF THE VERTICAL PATHS WITH

THE HORIZONTAL PATHS.

CROSSPOINT--A CROSSPOINT COMPRISES A SET OF CONTACTS THAT

OPERATES TOGETHER AND EXTENDS THE SPEECH AND SIGNAL WIRES OF THE

DESIRED CONNECTION. EACH CONNECTION IN A SPACE-DIVISION SWITCHING

NETWORK IS ESTABLISHED BY CLOSING ONE OR MORE CROSSPOINTS.

CROSSTALK--AN UNWANTED TRANSFER OF SIGNALS FROM ONE CIRCUIT TO

ANOTHER AS MAY OCCUR BETWEEN SWITCHING ELEMENTS OR CIRCUIT

WIRING.

C.W.A.--THE COMMUNICATION WORKERS OF AMERICA. THE C.W.A.

REPRSENTS 90 PERCENT OF THE UNIONIZED TELCO WORK FORCE.

DAY-TO-BUSY HOUR RATIO--THE RATIO OF THE 24 HOUR DAY TRAFFIC

VOLUME TO THE BUSY HOUR TRAFFIC VOLUME. IN SOME COUNTRIES THE

RECIPROCAL OF THIS RATIO IS USED.

D.D.D.--DIRECT DISTANCE DIALING. ALSO KNOWN AS ONE-PLUS DIALING.

DELAY SYSTEM--A SWITCHING SYSTEM IN WHICH A CALL ATTEMPT, WHICH

OCCURS WHEN ALL ACCESSIBLE PATHS FOR THE REQUIRED CONNECTION ARE

BUSY, IS PERMITTED TO WAIT UNTIL A PATH BECOMES AVAILABLE.

DIAL PULSE--THE SIGNALING PULSE WHICH IS FORMED BY THE

INTERRUPTION OF THE CURRENT IN THE DC LOOP OF A CALLING

TELEPHONE. SUCH INTERRUPTIONS ARE PRODUCED BY THE BREAKING OF THE

DIAL PULSE CONTACTS OF THE CALLING TELEPHONE SUBSET DURING THE

DIALING PROCESS.

DIAMOND-RING TRANSLATOR--AN ARRAY OF RING-TYPE INDUCTION COILS

ASSOCIATED WITH CODED WIRING IN SUCH A MANNER THAT THE

TRANSLATION OF DIRECTORY NUMBERS TO EQUIPMENT NUMBER OR VICE

VERSA CAN BE ACCOMPLISHED IN AN EXCHANGE. IT IS NAMED AFTER ITS

ORIGINATOR, T.L.DIMOND OF THE BELL TELEPHONE LABORATORIES.

DIRECT CONTROL--AN EXCHANGE CONTROL METHOD IN WHICH PULSES,

DIALED BY THE SUBSCRIBERS, CONTROL DIRECTLY THE ROUTE SELECTION

SWITCHES OF THE SYSTEM. FOR EACH DIGIT DIALED THE EQUIVALENT OF

ONE SET OF SELECTOR SWITCHES IS REQUIRED WITH THIS CONTROL

METHOD.

DIRECTOR--A CONTROL ELEMENT WHICH PROVIDES A MEASURE OF COMMON

CONTROL IN STEP-BY-STEP OR STROWGER EXCHANGES.

DISTRIBUTING FRAME--A STRUCTURE FOR TERMINATING THE WIRES OF A

TELEPHONE EXCHANGE IN SUCH A MANNER THAT CROSS-CONNECTIONS CAN BE

MADE READILY. EXAMPLES ARE THE MAIN DISTRIBUTION FRAME (MDF) AT

THE ENTRY OF AN EXCHANGE, INTERMEDIATE DISTRIBUTION FRAMES (IDF)

BETWEEN SECTIONS OF AN EXCHANGE, AND POWER DISTRIBUTION FRAMES

(PDF).

DISTRIBUTION STAGE--A SWITCHING STAGE BETWEEN A CONCENTRATION

STAGE AND OUTLETS AND SERVES AS A MEANS OF SELECTING TRUNKS TO

THE DESIRED TERMINATIONS.

DUV--DATA UNDER VOICE (AT&T SYSTEM). ELECTROMECHANICAL SWITCHING

SYSTEM--AN EXCHANGE SYSTEM IN WHICH BOTH THE SPEECH PATHS AND THE

CONTROL EQUIPMENT ARE SWITCHED BY ELECTROMECHANICAL

COMPONENTS--SUCH AS RELAYS, ROTARY SWITCHES, ETC.

ELECTRONIC SWITCHING SYSTEM--AN EXCHANGE SYSTEM IN WHICH AT LEAST

THE CONTROL EQUIPMENT IS COMPOSED OF ELECTRONIC CIRCUITS AND

COMPONENTS, GENERALLY OF A SOLID-STATE TYPE.

EMD SWITCH--THE SPEECH-PATH SWITCHING ELEMENT USED IN A SIEMENS

ROTARY SWITCHING SYSTEM. EMD IS AN ABBREVIATION OF

EDELMETALL-MOTOR-DREHWAHLER, WHICH TRANSLATES IN ENGLISH TO

NOBLE-METAL MOTOR SWITCH.

END OFFICE--A CENTRAL OFFICE OR CLASS 5 OFFICE.

ENTRAIDE--A SWITCHING SYSTEM INPHICH OUTLETS FROM A GIVEN

CONNECTING STAGE ARE CONNECTED TO INLETS OF THE SAME OR A

PREVIOUS STAGE. IN SUCH SYSTEMS CALLS MAY TRAVERSE A STAGE MORE

THAN ONCE. USUALLY THESE REENTERING LINKS ARE USED AS LAST CHOICE

PATHS AND THE RESULTING NETWORK IS HETEROGENEOUS. SUCH AN

ARRANGEMENT IS USED IN ITT'S PENTACONTA CROSSBAR SYSTEM.

ERLANG--THE UNIT OF TRAFFIC INTENSITY, WHICH IS MEASURED IN

CALL-SECONDS PER SECOND OR CALL-MINUTES PER MINUTE. ALSO, ONE

ERLANG EQUALS 3600 CALL-SECONDS PER HOUR. IT IS NAMED AFTER A. K.

ERLANG, THE DANISH ENGINEER AND MATHEMATICIAN WHO FIRST ADOPTED

IT.

ESS--ELECTRONIC SWITCHING SYSTEM.

EXCHANGE--ALL NUMBERS WITHIN A GIVEN THREE-DIGIT PREFIX AREA. CAN

ALSO BE USED TO DESCRIBE A GEOGRAPHICAL AREA THE SIZE OF A CITY.

FX--FOREIGN EXCHANGE CALLS. THE TERM APPLIED TO CALLS MADE TO A

CENTRAL OFFICE OTHER THAN THE ONE LOCATED IN THE CALLING CUSTOMER

AREA.

H.C. INSTRUMENT--AN ORDINARY TELEPHONE WITH NO EXTRAS.

I.D.E.W.--INTERNATIONAL BROTHERHOOD OF ELECTRICAL WORKERS. A

UNION THAT REPRESENTS SEVEN PERCENT OF ALL UNIONIZED TELEPHOE

WORKERS.

INTERSTATE--TELEPHONE SERVICE THAT CROSSES STATE LINES. SUCH

SERVICES COME UNDER THE JURISDICTION OF THE F.C.C.

INTRASTATE--TELEPHONE SERVICES THAT REMAIN WITHIN THE BOUNDARES

OF A STATE. SUCH SERVICES COME UNDER THE JURISDICTION OF THE

P.S.C.

JOINT PRACTICES--AN INTER-COMPANY GUIDE AKIN TO THE GENEVA RULES

OF WAR. THE J.P. COVERS SUCH THINGS AS INTERVALS, OFFERINGS, AND

PROCEDURES.

K.K.6--SIX-BUTTON TELEPHONE. THE STANDARD TELEPHONE FOUND IN MOST

OFFICES. THE K.K.6 CAN HANDLE FIVE LINES. THE SIXTH BUTTON IS

USED FOR HOLD.

LAYOUT CARD--SCHEMATIC DRAWINGS OF THE ELECTRICAL CIRCUITS

REQUIRED FOR A TELEPHONE INSTALLATION.

LINK (TRUNK) THE CONNECTION BETWEEN THE TERMINALS OF ONE SWITCH

AND THE TERMINALS ON A SWITCH OF THE NEXT STAGE CORRESPONDING TO

A SINGLE TRANSMISSION PATH.

LINK (ONE-WAY AND TWO-WAY)--A ONE-WAY LINK IS USED ONLY FOR THE

ESTABLISHMENT OF CONNECTIONS IN ONE DIRECTION, WHILE A TWO-WAY

LINK IS USED FOR THE ESTABLISHMENT OF CONNECTIONS IN EITHER

DIRECTION.

LINK SYSTEM--A SYSTEM IN WHICH: (1) THERE ARE AT LEAST TWO

CONNECTING STAGES; (2) A CONNECTION IS MADE OVER ONE OR MORE

LINKS; (3) THE LINKS ARE CHOSEN IN A SINGLE LOGICAL OPERATION;

AND (4) LINKS ARE SEIZED ONLY WHEN THEY CAN BE USED IN MAKING A

CONNECTION.

LOGIC FUNCTION--THE RELATIONSHIP OF TWO OR MORE BOOLEAN VARIABLES

AS EXPRESSED BY BOOLEAN ALGEBRA.

LOGIC GATES--ELECTRICAL OR ELECTRONIC CIRCUITS WHICH CONTROL THE

TRANSFER OF SIGNALS AND PRODUCE THE REQUIRED OUTPUTS FOR SPECIFIC

INPUT COMBINATIONS TO IMPLEMENT BOOLEAN LOGIC FUNCTIONS.

LOGIC (HARD-WIRED)--CONTROL LOGIC IN AN EXCHANGE, WHICH IS WIRED

IN CIRCUIT FORM.

LOGIC (SOFT-WIRED)--CONTROL LOGIC IN AN EXCHANGE, WHICH IS HELD

IN SOFTWARE COMPUTER PROGRAMS.

LONG DISTANCE--TECHNICALLY, ANY CALL THAT TERMINATES MORE THAN

SEVENTEEN MILES FROM THE SOURCE.

LONG LINES--A DIVISION OF AT&T RESPONSIBLE FOR THE DAY-TO-DAY

OPERATION OF THE LONG DISTANCE NETWORK. WHILE THE LOCAL TELCO

HANDLES ALL MAINTENANCE, LONG LINES DIRECTS OVERALL SUPERVISION.

LOOP DISCONNECT PULSING--SUBSET DIAL PULSING IN WHICH THE

SUBSCRIBER DC LOOP IS INTERRUPTED TO PRODUCE PULSES FOR SIGNALING

AN EXCHANGE.

MARKER--CIRCUITS WHICH INCORPORATE THE FUNCTION OF BUSY TESTING,

LOCATING AND FINALLY CONTROLLING THE ESTABLISHMENT OF A

PARTICULAR PATH THROUGH THE SWITCHING NETWORK.

MARKETING REP--THE SALES PEOPLE OF THE BELL COMPANIES. ALSO KNOWN

AS ACCOUNT EXECUTIVE.

MARKING--THE USE OF ELECTRICAL POTENTIALS AND GROUNDS AT CERTAIN

POINTS IN A SWITCHING NETWORK TO CONTROL ITS OPERATION.

MATRIX--A SIMPLE SWITCHING NETWORK IN WHICH A SPECIFIED INLET

(MATRIX ROW) HAS ACCESS TO A SPECIFIED OUTLET (MATRIX COLUMN) VIA

A CROSSPOINT PLACED AT THE INTERSECTION OF THE ROW AND COLUMN IN

QUESTION. A COMPLETE MATRIX IS ONE IN WHICH EACH INLET HAS ACCESS

TO EACH OUTLET, WHILE AN INCOMPLETE MATRIX IS ONE IN WHICH EACH

INLET MAY HAVE ACCESS TO ONLY SOME OF THE OUTLETS.

MEAN DELAY OF CALLS DELAYED--THE TOTAL WAITING TIME OF ALL CALLS

DIVIDED BY THE NUMBER OF DELAYED CALLS.

MESSAGE SWITCHING--A METHOD OF RECEIVING AND STORING A MESSAGE

FOR A MORE APPROPRIATE TIME OF RETRANSMISSION. WITH SUCH A

METHOD, NO DIRECT CONNECTION IS ESTABLISHED BETWEEN THE INCOMING

AND OUTGOING LINES AS IN THE CASE OF CIRCUIT SWITCHING.

MULTIFREQUENCY SIGNALING--SIGNALING BETWEEN SUBSCRIBERS AND THE

CENTRAL OFFICE THROUGH A COMBINATION OF AUDIO FREQUENCIES, AS

WITH PUSHBUTTON DIALING. ALSO, IN MANY CASES SIGNALING BETWEEN

EXCHANGES IS ACCOMPLISHED BY COMBINATIONS OF FREQUENCIES.

MULTIGROUP--A COMBINATION OF TWO OR MORE PCM MULTIPLEX CHANNELS.

NONLISTED NUMBERS--TELEPHONE NUMBERS THAT DO NOT APPEAR IN THE

DIRECTORY BUT THAT ARE AVAILABLE IF THE INQUIRER CALLS DIRECTORY

ASSISTANCE.

NONPUBLISHED NUMBERS--TELEPHONE NUMBERS NOT MADE AVAILABLE TO THE

PUBLIC. ALSO KNOWN AS SILENT NUMBERS.

OCCUPANCY--THE AVERAGE PROPORTION OF TIME THAT A TRAFFIC CARRYING

FACILITY IS BUSY.

PACKET SWITCHING--ESSENTIALLY THE SAME AS MESSAGE-SWITCHING.

PANEL-SWITCHING SYSTEM--A COMMON CONTROL ELECTROMECHANICAL

SWITCHING SYSTEM, WHICH WAS USED WIDELY IN THE U.S. PRIOR TO ITS

VIRTUAL REPLACEMENT BY CROSSBAR AND OTHER SYSTEMS. THE BANKS OF

SELECTORS TAKE THE FORM OF FLAT VERTICAL PANELS, FROM WHICH THE

NAME OF THE SYSTEM WAS DERIVED. SOME PANEL INSTALLATIONS ARE

STILL IN USE IN THE U.S.

PATH--A SET OF LINKS JOINED IN SERIES TO ESTABLISH A CONNECTION.

PATHS DIFFER IF ONE OR MORE LINKS DIFFER.

P.B.X.--PRIVATE BRANCH EXCHANGE. COMMONLY KNOWN AS A SWITCHBOARD.

MINI-CENTRAL OFFICE EQUIPMENT FOR BUSINESS CUSTOMERS WITH FROM 10

TO 2,000 TELEPHONES.

PRIMARY CENTER--A SWITCHING CENTER CONNECTING TOLL CENTERS, WHICH

CAN ALSO SERVE AS A TOLL CENTER FOR ITS LOCAL END OFFICES. IN THE

U.S. IT IS DEFINED AS A CLASS 3 OFFICE.

PRIVATE AUTOMATIC BRANCH EXCHANGE (PABX)--A PRIVATE AUTOMATIC

TELEPHONE EXCHANGE WHICH PROVIDES FOR THE CONNECTION OF CALLS

GOING TO AND COMING FROM THE PUBLIC TELEPHONE NETWORK (USUALLY A

CENTRAL OFFICE EXCHANGE) AS WELL AS INTRAEXCHANGE CALLS BETWEEN

THE SERVED EXTENSIONS.

PROBABILITY OF DELAY--THE PROBABILITY THAT A CALL ATTEMPT, IF

OFFERED, CANNOT BE COMPLETED IMMEDIATELY.

PROBABILITY OF LOST CALLS (PROBABILITY OF LOSS)--THE PROBABILITY

THAT A CALL ATTEMPT, IF OFFERED, WILL BE LOST.

PROGRAM STORE--THE MEMORY SECTION OF A STORED PROGRAM CONTROL

SWITCHING IN WHICH SEMI-PERMANENT INSTRUCTIONS AND TRANSLATIONS

ARE CONTAINED. THESE ARE FED TO THE CENTRAL PROCESSOR TO PERMIT

IT TO PROVIDE STORED PROGRAM CONTROL.

PUBLIC SWITCHED NETWORK--ANY SWITCHING SYSTEM THAT PROVIDES

CIRCUIT SWITCHING FACILITIES FOR USE BY THE PUBLIC. TELEPHONE,

TELEX, TWX, AND BROADBAND SWITCHED NETWORKS ARE THE PUBLIC

SWITCHED NETWORKS IN THE U.S.

PULSE AMPLITUDE MODULATION (PAM)--A FORM OF PULSE MODULATION IN

WHICH A NUMBER OF CHANNELS ARE MULTIPLEXED BY TIME SAMPLING, BUT

ONE IN WHICH THE PULSE AMPLITUDES VARY IN ACCORDANCE WITH THE

AMPLITUDE OF THE ANALOG SIGNAL LEVELS.

LATA: LOCAL ACCESS TRANSFERENCE AREA. SOME PEOPLE WHO LIVE IN LARGE CITIES

OR AREAS MAY BE PLAGUED BY THIS PROBLEM. FOR INSTANCE, LET'S SAY YOU LIVE IN THE

215 AREA CODE UNDER THE 542 PREFIX (AMBLER, FORT WASHINGTON). IF YOU WENT TO

DIAL IN A BASIC METRO CODE FROM THAT AREA, FOR INSTANCE, 351-0100, THAT MIGHT

NOT BE COUNTED UNDER UNLIMITED LOCAL CALLING BECAUSE IT IS OUT OF YOUR LATA.

FOR SOME LATA'S, YOU HAVE TO DIAL A '1' WITHOUT THE AREA CODE BEFORE YOU CAN

DIAL THE PHONE NUMBER. THAT COULD PROVE A HASSLE FOR US ALL IF YOU DIDN'T

REALIZE YOU WOULD BE BILLED FOR THAT SORT OF CALL. IN THAT WAY, SOMETIMES, IT IS

BETTER TO BE SAFE THAN SORRY AND PHREAK.

THE CALLER LOG: IN ESS REGIONS, FOR EVERY HOUSEHOLD AROUND, THE PHONE

COMPANY HAS SOMETHING ON YOU CALLED A CALLER LOG. THIS SHOWS EVERY SINGLE NUMBER

THAT YOU DIALED, AND THINGS CAN BE ARRANGED SO IT SHOWED EVERY NUMBER THAT WAS

CALLING TO YOU. THAT'S ONE MAIN DISADVANTAGE OF ESS, IT IS MOSTLY COMPUTERIZED

SO A NUMBER SCAN COULD BE DONE LIKE THAT QUITE EASILY. USING A DIALUP IS AN EASY

WAY TO SCREW THAT, AND IS SOMETHING WORTH REMEMBERING. ANYWAYS, WITH THE CALLER

LOG, THEY CHECK UP AND SEE WHAT YOU DIALED. HMM... YOU DIALED 15 DIFFERENT 800

NUMBERS THAT MONTH. SOON THEY FIND THAT YOU ARE SUBSCRIBED TO NONE OF THOSE

COMPANIES. BUT THAT IS NOT THE ONLY THING. MOST PEOPLE WOULD IMAGINE "BUT WAIT!

800 NUMBERS DON'T SHOW UP ON MY PHONE BILL!". TO THOSE PEOPLE, IT IS A NICE

THOUGHT, BUT 800 NUMBERS ARE PICKED UP ON THE CALLER LOG UNTIL RIGHT BEFORE THEY

ARE SENT OFF TO YOU. SO THEY CAN CHECK RIGHT UP ON YOU BEFORE THEY SEND IT AWAY

AND CAN NOTE THE FACT THAT YOU f***!ED UP SLIGHTLY AND CALLED ONE TOO MANY 800

LINES.